DragStrip Plus

User's Manual

Version 4.XX

# Table Of Contents

- Table Of Contents

License Notices

Copyright Notice

Limited Warranty

DRAGSTRIP PLUS Disclaimer

WARNING

Limitation Of Liability

Trademarks

DRAGSTRIP PLUS History & Credits

History And Credits

Introducing DRAGSTRIP PLUS

About DRAGSTRIP PLUS

Using This Manual

Using Manual

Typographical Conventions

System Requirements

Hardware Requirements

Software Requirements

Chapter One:  Getting Started

Program Installation

Program Configuration

Setting Your Password

Chapter Two:  Using DragStrip Plus

Quick Tour Of DragStrip Plus

DragStrip Plus Hot Keys

Chapter Three:  Using An Analysis Mode

Choosing The Correct Analysis Mode

Quick Compute Mode

Detailed Analysis Mode

Dyno Approach Mode

Chapter Four:  Using Quick Analysis

The Quick Compute Mode

Chapter Five:  Entering Your Combination

Analysis Mode

Driver's Weight

Vehicle's Weight

Weight Change

Nitrous Oxide Use

Amount Of Nitrous Oxide Horsepower

Stall Point

Exhaust System

Tire Height

Tire Selection

Traction

DragStrip Plus' Traction Definitions

Name Your Vehicle

The E.T. Results Screen

Making Changes To Your Vehicle Combination

Header Tubes

Header Tube Size

Exhaust Tube Size

Engine Size

Number Of Engine Cylinders

Engine Manufacturer

Engine Type

Transmission Selection

Number Of Transmission Gears

Transmission Gear Ratios

Finish Line Gear

DRAGSTRIP PLUS Camshaft Type

Total Camshaft Duration

Camshaft Duration At .050"

Valve Lift

Camshaft Lobe Centers

Camshaft Overlap

Cam Installation Timing

Intake Valve Size

Exhaust Valve Size

Rocker Arm Ratio

Carburetor Type

Carburetor Size or CFM

Air Induction

Superchargers

Turbochargers

Turbocharger / Supercharger Boost

Number of Turbochargers

Intercooler Selection

Manifold Selection

Ignition Selection

Piston Selection

Engine Compression

Cylinder Head Selection

Cylinder Head Porting

Engine Fuel

Fuel Pump

Engine Quality

Elevation

Air Temperature

Relative Humidity

Barometric Air Pressure

Engine Temperature

Vehicle Body Style

Body Style Frontal Area

Driver Experience

Give Your Vehicle A Name

E.T. Results Screen

Chapter Six:  E.T. Discrepancy Errors

E.T. Discrepancy

Chapter Seven:  Analysis And Ratings

Engine Analysis And Ratings

Chapter Eight:  Combination Advice

DRAGSTRIP PLUS Combination Advice

Chapter Nine:  Combination Changes

Vehicle Data Screen

Engine Data Screen

Chapter Ten:  Using Dyno Mode Analysis

Using The Dyno Approach Mode

Engine Horsepower

Gross And Net Horsepower

Driver's Weight

Vehicle's Weight

Weight Change

Nitrous Oxide Use

Amount Of Nitrous Oxide Horsepower

Nitrous Horsepower Usage

Stall Point

Exhaust System

Tire Height

Tire Selection

Traction

DragStrip Plus' Traction Definitions

Naming Your Vehicle

Appendix  A

Changing Computer's Shift Point

Transmission Slippage Change

Appendix  B

Unfinished Vehicle In Database

Vehicle Database

Vehicle Database Information

Appendix  C

Vehicle Weights

Appendix  D

Parameter File and Ratings File Documentary

Appendix  E

Setting Your Dial-In

Appendix  F

Upcoming New Products

# License Notices

## Copyright Notice

This manual and other accompanying written and disk-based notes and specifications, and all referenced and related program files accompanying this document, are copyrighted 1992-1995 by Revolutionary Concepts, Inc.

No part of this manual may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language (natural or computer), in any form or by any means, without the prior written permission of:

Revolutionary Concepts, Inc.

P.O. Box 690048

Houston, Texas  77269-0048

E-Mail: dragplus@rconcepts.com

You are granted a limited license to use the software described in this manual.

Information in this manual is subject to change without notice and does not represent a commitment on the part of Revolutionary Concepts, Inc.

Revolutionary Concepts, Inc. may make improvements and/or changes in this manual and/or in DRAGSTRIP PLUS (computer program) at any time without obligation of any kind to the purchaser or owner of DRAGSTRIP PLUS.

DRAGSTRIP PLUS User's Manual Version 4.0a  Copyright © 1995, by Revolutionary Concepts, Inc.  All rights reserved. Cover designed by Jeff Williams, Houston, Texas.  DRAGSTRIP PLUS is exclusively distributed in the United States by Martel Brothers Racing Products.

FOR ORDERS CONTACT:	MARTEL BROTHERS RACING PRODUCTS

5328 E. 5th Street

Katy, Texas  77493

Phone: 1-800-882-RACE  or  (713) 391-7772   Fax: (713) 391-8519

FOR TECHNICAL SUPPORT:	Revolutionary Concepts, Inc.  Phone: (713) 859-RACE (collect return calls only) or

E-Mail: dragplus@rconcepts.com

## Limited Warranty

With respect to the physical diskette and physical documentation, Revolutionary Concepts, Inc. warrants same to be free of defects in materials and workmanship for a period of 90 days from the date of purchase.  In the event of notification within the warranty period of defects in material or workmanship, Revolutionary Concepts, Inc. will replace the defective diskettes or documentation.  The remedy for breach of this warranty shall be limited to replacement and shall not encompass any other damages, including but not limited to loss of profit, and special, incidental, consequential, or other similar claims.

## DRAGSTRIP PLUS Disclaimer

USE OF DRAGSTRIP PLUS SIGNIFIES AGREEMENT BY USER TO ALL THE TERMS AND CONDITIONS OF THIS DISCLAIMER.  USERS OF DRAGSTRIP PLUS MUST ACCEPT THIS DISCLAIMER OF WARRANTY: REVOLUTIONARY CONCEPTS, INC. MAKES NO WARRANTY OF ANY KIND, EXPRESSED OR IMPLIED, INCLUDING WITHOUT LIMITATION, ANY WARRANTIES OF MERCHANTABILITY AND/OR FITNESS FOR A PARTICULAR PURPOSE.  REVOLUTIONARY CONCEPTS, INC. ASSUMES NO LIABILITY FOR DAMAGES, DIRECT OR CONSEQUENTIAL, WHICH MAY RESULT FROM THE USE OF DRAGSTRIP PLUS.

## WARNING

Engine combinations designed and/or built by use of DRAGSTRIP PLUS or contained in DRAGSTRIP PLUS are designed for competition purposes.  Accordingly, use of said DragStrip Plus, or modification to or construction of a vehicle and/or engine combination for those purposes may create dangerous conditions which could cause bodily injury, and the buyer and/or users hereby expressly assume all risk associated with any such modifications.

## Limitation Of Liability

In no case shall Revolutionary Concepts, Inc. liability exceed the license fees paid for the right to use the Licensed Software or one hundred and forty-nine dollars ($149.95), whichever is less.

## Trademarks

MS-DOS is a registered trademark of Microsoft Corporation

Compaq is a registered trademark of Compaq Computer Corporation.

DRAGSTRIP PLUS is a registered trademark of Revolutionary Concepts, Inc.

IBM is a registered trademark of International Business Machines Corporation.

# DRAGSTRIP PLUS History & Credits

## History And Credits

DRAGSTRIP PLUS has been under development using a variety of languages since 1985. DRAGSTRIP PLUS went on sale in December, 1992, and the first shareware version was produced in August, 1995.

DRAGSTRIP PLUS does not rely on conventional formula-based mathematics, but instead uses sophisticated algorithms similar to human-like logic and reasoning as its primary approach to problem solving and data analysis.

PAUL I. WILLIAMS

Program Concept, E.T. logic, Quick Basic programming

FREDERICK L. BRADEN

C-language programming, program layout, Internet services

RON VOTAW

Animated Computer graphics

LAWRENCE CONLEY

Senior Technical Advisor

JARVIS W. THOMPSON, PAT McCHAN, PAUL LATOUR, and BOB GULITTI

Drag Racing Technical Expertise

## Introducing DRAGSTRIP PLUS

## About DRAGSTRIP PLUS

INTRODUCING A NEW ERA IN DRAG RACING:  THE AGE OF THE COMPUTER DESIGNED ENGINE!

WHY YOU NEED DRAGSTRIP PLUS

DRAGSTRIP PLUS signals the arrival of a whole new era in drag racing...  the age of the computer designed combination.

DRAGSTRIP PLUS takes all the guesswork out of your engine design process.  Instead of trial and error, you get a clear statement of your combination's strengths and weaknesses.  You are able to build it right the first time, which saves you time and money.

DRAGSTRIP PLUS lets you know before reaching the track what your combination is going to do when it gets there.  No more embarrassment, no more unpleasant surprises, no more swapping and changing parts trying to get your vehicle to perform up to your expectations!

In the never ending battle to stay ahead of the competition, DRAGSTRIP PLUS is your winning edge, your "ace in the hole".  Here's how DRAGSTRIP PLUS can work for you:

IF YOU "GRUDGE" RACE, HERE ARE TWO POPULAR USES OF DRAGSTRIP PLUS:

1.	Use DRAGSTRIP PLUS to find out how fast your competition is running, then use that knowledge to beat them!

Then, as fast as your competition tries to improve, use DRAGSTRIP PLUS to discover how their performance has been helped (or hurt!) by the changes they make to their combinations, even before your opponents can find out themselves!

They'll never know how you seem to know more about their cars than they do --nor why you are able to race and beat them with such confidence!

TIP:	Use DRAGSTRIP PLUS' "Quick Compute" mode to predict your competition's performance if you are only familiar with your opponents' basic combination and finish-line MPH (or RPM).

If you are thoroughly familiar with your opponents' combination, use DRAGSTRIP PLUS' "Detailed Analysis" mode to get a comprehensive analysis of your competition's performance.

NOTE:	Both of these techniques are explained in greater detail later on in this manual.

2.	Use DRAGSTRIP PLUS at home to prevent your competition from discovering your E.T.s at the track.  Since you already know what your vehicle will do when you get to the track, there is no need to blow your cover.

Instead, you can set up races for yourself by cruising down the track at part throttle, pretending something is wrong with your engine.

Then when your competition jumps you and wants to race, you can surprise them as your engine miraculously "fixes" itself and runs its best E.T. of the day, just as predicted in advance by DRAGSTRIP PLUS.

IF YOU ARE AN ENGINE BUILDER:

1.	Use DRAGSTRIP PLUS to design original engines of incredible power and unparalleled performance.  Or if you prefer, use DRAGSTRIP PLUS to build engines of ordinary performance, but possessing new and unique performance capabilities, such as enhanced streetability or custom shift point.

2.	If you build race engines commercially, you can deliver your engines with a printout of DRAGSTRIP PLUS' engine combination analysis, with each analysis based upon your customer's likely vehicle setup.

Even if your customers are paying for dyno work and are expecting a dyno printout to accompany their engine, you can still include a printout of DRAGSTRIP PLUS' combination advice.

You will discover that your customers will come to value the computer aided advice even more than they do the dyno sheets --after all, DRAGSTRIP PLUS' primary goal is fastest possible E.T.s, in contrast to dyno's focus on maximum possible engine horsepower.

IF YOU BRACKET RACE:

DRAGSTRIP PLUS was primarily designed to help the performance enthusiast and combination engine builder.  But DRAGSTRIP PLUS can be significant aid to the bracket racer as well.  Here how:

1.	If you wish to drop your vehicle into the next fastest bracket racing category, use DRAGSTRIP PLUS to work out the easiest and most practical method to accomplishing this for the least amount of money spent.

2.	You can use DRAGSTRIP PLUS to precisely set your track "dial in", based upon your previous dial-ins.

After you have gained a thorough operational knowledge of DRAGSTRIP PLUS, consult Appendix E for additional information on this technique.

Don't get left behind...  No matter what race application, let DRAGSTRIP PLUS work for you.  Don't go racing without it!

DRAGSTRIP PLUS, the intuitive racing software, is everything you'll ever need in racing software!  Here's a brief summary of its major features:

Specialized Equipment.  DRAGSTRIP PLUS does not need dynometer, flowbench, air mass meter, or other specialized technical data for program input.  You need only an IBM or compatible computer.

Vehicle Specs.  Does not need your vehicle's current E.T., MPH, finish-line RPM, or horsepower to generate its results.  Needs only the specs of your vehicle and engine's parts.

Reports.  Provides specific reports detailing your engine's performance, including suggestions on how best to improve its performance.  Forecasts with uncanny accuracy the precise E.T. your vehicle will run once you make its suggested changes.

Calculations.  Automatically calculates the correct gear, stall, shift point, and carburetor size for each combination's optimum performance, and computes its gross and net horsepower.

Database.  Stores up to 115 vehicles in its database for quick and easy retrieval.

Context-Sensitive Help.  DRAGSTRIP PLUS gives you full-screen detailed help no matter where you are in the program.  The context-sensitive help facility always knows "what you are doing" and displays applicable help on the current task.

# Using This Manual

## Using Manual

This manual provides general information and specific instructions you'll need to use the many features of DRAGSTRIP PLUS.  It includes a tutorial to lead you through four example combinations.

## Typographical Conventions

To avoid confusion, we've used distinct conventions throughout this manual to represent keys you should press and information you should type.

·	A key you should press appears like this:  <ENTER>.

·	A key combination like <ALT><F1> means you should hold down the <ALT> key while you press the <F1> key.

·	The spacebar key appears like this:  <SPACE>

·	A numeric zero appears like this: 0.  An uppercase letter "oh" appears like this: O.

·	A command you need to type exactly as shown appears like this:

copy a:*.*

# System Requirements

## Hardware Requirements

Minimum: COMPAQ, IBM, or compatible personal computer or laptop with MS-DOS Version 2.1 or higher operating system, 640K RAM memory, 3 1/2" high density floppy disk drive, hard drive with 1 mb free disk space; CGA, EGA, VGA, Monochrome, or LCD monitor.

## Software Requirements

DISKCHK	RACE0xx.RTS

INSTALL.BAT	RACE1xx.HLP

HARDDRIVE.TXT	RACE2xx.HLP

MANUAL.DOC	RACE3xx.HLP

MANUAL.TXT	RACE4xx.HLP

RACE.BAT	RACE007.DB

RACE.CFG	RACE007.DBK

RACE.DOC	RACE4xx.EXE

RACE0xx.PRM	README.TXT

RACEPRM.DOC	WHATSNEW.TXT

Animation Software is provided on a second disk as an optional display feature and is not required to run DRAGSTRIP PLUS.  A VGA monitor is  are required to use Animation Software:

AAPLAY.EXE     DP3.FLI

# Chapter One:  Getting Started

## Program Installation

Installation and setup of the program is simple.

FLOPPY DISK INSTALLATION (IF YOU DO NOT HAVE A HARD DRIVE)

1.	Boot your computer system with your own DOS diskette (or by whatever means you normally 	use to boot it).

2.	Place the DRAGSTRIP PLUS program disk in your floppy disk drive.

3.	Make DRAGSTRIP PLUS the current drive:

Type: a: (or) b:  {whichever drive contains the DRAGSTRIP PLUS program disk}

Press: <ENTER>

Type: race

Press: <ENTER>

4.	To exit DRAGSTRIP PLUS, press <F10> from anywhere in the program, then press     number <4> to exit to DOS at the Main Menu.

HARD DISK INSTALLATION

1.	Start your computer.

2.	After your computer boots, place the DRAGSTRIP PLUS program disk into your floppy disk 	drive.

3.	If any custom menus or application software loaded when you started your computer, exit and return to DOS.

4.	Make DRAGSTRIP PLUS the current drive:

Type: a: (or) b:  {whichever drive contains the DRAGSTRIP PLUS program disk}

Press: <ENTER>

Type: install

Press: <ENTER>

NOTE:	DRAGSTRIP PLUS will be installed on your hard drive (drive C:) in a directory it creates named RACE.  If an older version of DRAGSTRIP PLUS already exists in this directory, the old version will be replaced by the latest version of DRAGSTRIP PLUS.  However, your existing car combinations (if any) will be preserved and will not be overwritten.

5.	To exit DRAGSTRIP PLUS, press <F10> anywhere in the program, then press number <4> to 	exit to DOS at the Main Menu.

## Program Configuration

DRAGSTRIP PLUS may be executed from either the original floppy, or a hard drive, whichever you prefer.  If you choose to install DRAGSTRIP PLUS on your hard drive (recommended), all program files on the original floppy must be copied to the same subdirectory on your hard drive (done automatically by the Install program), and the program must be executed from this subdirectory.

NOTE:	It is important that the user read and understand all Help information provided by the program for its required input data.  On-line Help is accessible throughout the program by pressing the <F1> key, or by consulting this manual.

## Setting Your Password

Password protection is provided with DRAGSTRIP PLUS as an optional feature insure the security of our user's valuable engine combinations.

If you so choose, it is easy to enable DRAGSTRIP PLUS' password protection. Just start DRAGSTRIP PLUS, and when the program's main menu appears --the main menu is the first screen to appear when you start the program --select the "SETUP" option.  Enter a password that you will remember, as instructed.  The program will then ask you to confirm your password by entering it again.

From now on, the program will ask you for your private password to gain access.  If for any reason you wish to reverse this procedure, select the "SETUP" option at the main program menu, enter your current password, then select the "new password" option.  But instead of entering a new password, just press the <ENTER> key (twice).  This action will remove your password and will make it no longer necessary to enter a password to gain access to the program.

SPECIAL NOTE:	If you forget your password, just leave us a message with your name and phone number together with a likely time you can be reached, and we will do our best to assist you.

# Chapter Two:  Using DragStrip Plus

## Quick Tour Of DragStrip Plus

If you are new to DRAGSTRIP PLUS, you will have better results if you first experiment with some of the sample cars included with the program rather than loading the program and promptly trying to enter your own vehicle.

Four example combinations are included on your DRAGSTRIP PLUS program diskette in the vehicle database as a learning aid.

Like the other vehicles included with DRAGSTRIP PLUS, these example combinations are not fictitious, but are "real world" cars based in Houston, Texas, that perform almost precisely the E.T.s, MPHs, and shift points displayed by the program.

We have included these sample cars in your vehicle database as a valuable example of how to make the best use of DRAGSTRIP PLUS.  In particular, we will concentrate on helping you develop the thought processes necessary to design engine combinations that are far superior to the combinations developed by the more casual users of DRAGSTRIP PLUS!

Let's start our demonstration by loading the first example car.  If you do not already have DRAGSTRIP PLUS loaded, start the program by typing "RACE" at the DOS prompt (see prior sections in this manual for help in installing the program if necessary), enter your password (if applicable), then when the "Mode Choice Selection" screen appears, press <F4> to enter the vehicle database.

SELECT YOUR CHOICE OF PROGRAM ANALYSIS MODES:

1) `Quick Compute'	You supply only parts related to the vehicle plus

its finish-line MPH or RPM, and DragStrip Plus

will compute its E.T.  Quick but highly accurate!

2) `Detailed Analysis'	You supply all engine and vehicle parts, and

DragStrip Plus will compute everything: the

vehicle's finish-line MPH, RPM, and E.T., plus

provide a comprehensive engine analysis.

3) `Dyno Approach'	You supply the vehicle's gross or net horsepower,

and DragStrip Plus will compute its E.T. and MPH.

F1=HELP       F3=SOUND<ON>        F4=DATABASE        F8=ET SCREEN        F10=END

Press <F4> to enter the vehicle database.

File #	Vehicle Name	Elapsed Time	MPH	[Current: #12]

-----------------------------------------------------------

1	EXAMPLE CAR (start)	14.03	99.5

2	EXAMPLE CAR (improved)	13.19	102.1

3	EXAMPLE CAR (on NOS)	12.32	108.7

4	EXAMPLE CAR (best)	10.97	122.5

5	Pro-mod combination!	6.74	203.9

6	GT Mustang (stock)	15.21	91.7

7	The SunChaser Camaro	11.62	122.3

8	John's '70 Camaro	12.99	109.9

9	Matt's street Z28	13.85	105.4

10	Big Block Super Car	9.69	138.9

11	Radical Small Block	9.73	142.5

12	Heavy Chevy Pickup	13.51	95.3

13	528 CID T-Bird!	8.81	158.0

14	Big Block Camaro	12.45	114.0

15	Terrible Combination	11.91	121.9

16	Small block Nova	12.08	110.9

17	Larry's 8-sec. Monza	8.83	155.7

<ESC=EXIT> <F4=SELECT> <F5=COPY> <F6=SWAP> <F7=PRINT> <F9=DELETE> <ENTER=START OVER>

Press <F4> again to signal you wish to load a vehicle, then enter the record number (which is #1) of the first example car, and press <ENTER>.

COMPUTER ENGINE ANALYSIS RESULTS

Vehicle: #1  EXAMPLE CAR (start)	Date: 05-03-1994  18:37:41  3.00

MPH	Gear	Tire	RPM	Slip%

99.5	4.11	27.0	5605	10.0

Best possible ET, uncapped:	12.68	Using optimum Stall would

lower your estimated

Est	60 foot ET:	(  32.1 mph)	2.05	1/4 mile E.T. to:	13.58  sec.

Est	1/8 mile ET:	(  80.9 mph)	8.69

Est	1/4 mile ET:	(  99.6 mph)	14.03	Optimum stall:	4067

Optimum 1/4 mile Gear:	4.69

Biggest E.T. losses (in order):	or  use tire height of:	23.6"

Stall	- 0.44	Optimum 1/8 mile Gear:	5.83

Traction	- 0.42	or  use tire height of:	19.0"

Gear	- 0.32

------------------	Gross Horsepower(aprox):	376.00

Total Losses: 1.18 seconds	Net Horsepower (exact):	279.03

All calculations based upon current Elevation of 100 feet.

<F1=HELP> <F2=NEW VEHICLE> <F4=DATABASE> <ENTER=CONTINUE> <F7=PRINT> <F10=QUIT>

Now the fun starts. Looking at the "E.T. Results Screen", we notice that this vehicle is only running in the low 14's at approximately 100 MPH:

Est. 1/4 mile E.T.:  14.03 at 99.5 MPH

NOTE:	The exact E.T. displayed by DRAGSTRIP PLUS changes slightly from time to time as new versions are released and improvements to its computing logic are made.

The number one vehicle loss listed is "not enough stall", followed closely by "not enough traction" and "not enough gear".  Based upon this observation, our initial reaction might well be to increase the amount of stall and thus "improve" the combination significantly in this manner.

But why mess up a perfectly good street car?  DRAGSTRIP PLUS reports that the optimum stall needed for this combination is roughly 4100, with roughly 4.56 as the optimum gear.  Why get rid of a 2500 stall and 4.11 gears, when there might be a better way to improve this combination, utilizing the existing stall and gear?

It turns out that there is a far better way to dramatically increase the performance of this vehicle than by changing the stall and gear.  Press <ENTER> to continue to the next screen, the "Engine Statistics Screen".  Notice that this vehicle's shift point is above 6400, while the engine's powerband is almost 2400 RPM wide.

ENGINE STATISTICS SCREEN

Vehicle: #1  EXAMPLE CAR (start)	Date: 05-03-1994

Combination / Design Rating:	75.8	Optimum Carb CFM: 512 - 671

Engine-to-Vehicle Matchup Rating:	64.5	Opt Primary Header Len: 39.3"

Engine Overall Power Rating:	42.6

Computer Estimated Shift Point:	6451	Shift MPH in 1st gear:	46.1

Estimated Engine Powerband Width:	2384	2nd gear:	64.5

3rd gear:	113.5

Engine Size in CID :	355

Computer Predicted HP per CID:	1.02

Computer Est. of Lost Exhaust HP:	16.8

Nr. of Badly Mismatched Parts:	1

Highest Rated Engine Part(s):	Total Duration	Rating:	115.4

(100 = Perfect)	Header size	Rating:	114.4

Lowest Rated Engine Part(s):	Duration @.050"	Rating:	88.1

Port job	Rating:	67.4

<F1=HELP> <F2=NEW VEHICLE> <F3=ADVICE> <ENTER=CONTINUE> <F8=BACK> <F10=QUIT>

While these numbers aren't so bad for a street car, what if they could be improved?  Instead of increasing the amount of stall and gear, why not lower and widen the engine's powerband if possible?

COMBINATION PART RATINGS

Vehicle: #1  EXAMPLE CAR (start)	Date: 05-03-1994

RATING OF 100 = PERFECT:	Computer Suggestion(s):

-------------------------------	-------------------------------------

Total Duration	115.4	Advance CAM -or- add more gear

Header size	114.4

Head type	105.4	Camshaft profile needs less Total

Carburetor CFM	103.7	(advertised) Duration

Intake valve size	103.0

Exhaust valve size	101.9	Try increasing Cam lobe separation

Cam Overlap	101.4

Overall Camshaft Profile	100.2	Consider upgrading to a Solid-lift CAM

Compression	98.3

Exhaust pipe size	97.4	Try smaller headers. (Press F1 for Help)

Valve lift	95.7

Intake Manifold	94.4	Increase fuel pump delivery!

Cam type	94.1

Fuel octane	90.3	Better head port job BADLY needed

Duration @.050"	88.1

Port job	67.4

<F1=HELP>   <F4=DATABASE>   <F7=PRINT>   <ESC=EXIT>   <ENTER=CONTINUE>   <F10=QUIT>

Now press <F3> to enter the "Combination Advice Screen".  Based upon the suggestions displayed on this screen, we decide to spend $500 or so and upgrade the engine by making the following changes:

1.	Smaller headers (1 5/8" verses 1 3/4")

2.	Better CAM (more valve lift for less total duration, wider lobe centers)

3.	Advancing the new CAM four degrees at installation

4.	Mildly porting the heads

5.	Better tires & traction (McCrearys and "Good" traction instead of Pro-Tracs and "Satisfactory" traction)

6.	Cooling the engine completely before racing (instead of entering the staging lanes with an already warm engine --see HELP in this manual under this topic, "Engine Temperature")

ENGINE DATA SUMMARY

Vehicle: #1  EXAMPLE CAR (improved)	Date: 05-03-1994

1) Header Tube size:   1 5/8	E) Ex.Valve Size:	1.50

2) Exh. Pipe size:	2 1/2	F) Rocker Ratio:	1.5 : 1

3) Engine CID:	355 Chevy	G) Carburetor:	Holley

4) Transmission:	Turbo 350	H) Carb. CFM:	650

1st Gear:	2.46	J) Air Induction:	Ram/Cowl Air

2nd Gear:	1.76	K) Boost:	N/A

3rd Gear:	1.00	L) InterCooler:	N/A

4th Gear:	N/A	M) Manifold:	Lo-rise single plane

5th Gear:	N/A	N) Ignition:	Stock Electronic

5) Finish Line Gear:   3rd	P) Pistons:	Cast

6) Camshaft:	Hydraulic	Q) Compression:	8.5 : 1

7) Tot. Duration:	296	R) Head Type:	Open-chamber iron

8) Dur. @ .050":	245	S) Port Job:	Basic/Port Matched

9) Valve lift:	.535"	T) Fuel:	93 octane Super Unleaded

A) Lobe Centers:	110	U) Fuel pump:	Stock

B) Cam Overlap:	65	V) Engine Quality:Excellent(Most cars)

C) Cam Timing:	4 Advanced	W) Driver Skill:	Good

D) In. Valve Size:	1.94

<F1=HELP> <F2=NEW VEHICLE> <F4=DATABASE> <ENTER=CONTINUE> <F8=BACK> <F10=QUIT>

Normally, to make these changes you would need to press <ENTER> until you come to the "Engine Data Summary Screen", then make the changes one at a time.  But there is no need to do so, as example car #2 in the vehicle database already has these changes made to it.

Return to the vehicle database by pressing <F4>.  Load example vehicle #2 from the database, then notice the results of our changes:

Est. 1.4 mile ET:  13.19 at 102.1 MPH

Wow!  What did we do specifically that resulted in an E.T.  improvement like this?  Our changes substantially lowered the new combination's shift point to just over 5900 RPM, while improving the engine's already generous powerband to nearly 2600 RPM.  Also the Engine-to-Vehicle Matchup rating now stands at 85, verses a miserable 64 rating before.

In addition, this new combination produces 18 more horsepower.  But keep in mind that adding horsepower was not the main purpose of making the changes we that made.  Lowering and widening the engine's powerband was the primary, indeed only, goal of the combination changes we made!

This is a very important point.  Unlike a mechanical dyno, DRAGSTRIP PLUS does not primarily seek to boost your engine's horsepower output, but rather to get your entire vehicle as a package to go faster, in a less "radical" form if possible, EVEN IF THIS MEANS REDUCING YOUR ENGINE'S OVERALL HORSEPOWER OUTPUT!

What if we want to go still faster?  Is there any thing else we can do to make our 3400 lb. small block street car substantially faster still?

Yes, there is. And since, fortunately or not, many street cars have nitrous oxide capability, we decide we don't wish to be left behind. So let's redesign our combination with nitrous oxide in mind. Here's the new changes:

1.	Forged flat top pistons (instead of cast dished pistons)

2.	Compression ratio of 9.5 : 1, instead of 8.5 : 1, as a result of the piston change

3.	Hi-performance after market ignition system

4.	Electric fuel pump, 125 gpm  (very important!)

5.	Adding octane booster or mixing in racing fuel in with our Super Unleaded gas, to allow Nitrous use without having to sharply retard the engine (with nitrous, you may not hear cylinder detonation in time to prevent damage)

6.	A 150 (gross) horsepower Nitrous Oxide kit

7.	"Satisfactory" traction instead of "Good" traction, allowing for the increased engine torque and tire spinnage due to the addition of nitrous..

These changes have already been made for us in your vehicle database.  Load example vehicle #3 from the vehicle database (by pressing <F4> again), and notice the results of our work:

Est. 1/4 mile ET:  12.32 at 108.7 MPH

Notice that this combination's shift point has declined still further to under 5800 RPM, while the engine's powerband has widened to a whopping 2800 RPM!  However, while this vehicle's E.T. performance is great, it is still not particularly outstanding compared to other state-of-the-art street machines running out there.  So what can we do to improve this combination still further?

At this point, we decide to spend another $1000 - $2600 or so and build ourselves a top class (but still relatively cheap) Pro-Street machine.  Here's the new changes:

1.	Reduce vehicle weight to 3100 lbs., down from 3400

2.	Increase transmission stall to 3200 RPM (up from 2500)

3.	Add free-flowing mufflers and 3 inch exhaust (verses turbo type mufflers and 2 1/2" exhaust before)

4.	Add Slicks, improving traction to "Good"

5.	Change to a Solid-lift Camshaft (more valve lift for the same camshaft total duration)

6.	Camshaft installed straight up instead of advanced

7.	Closed-chamber production heads

8.	2.02" intake and 1.60" exhaust valves

9.	"Thorough" port job on the new heads, plus intake manifold and gasket port matching

10.	10:1 Compression (result of head change)

11.	100-105 octane racing fuel

12. 	Change to a medium-rise single plane manifold (such as Victor Jr.)

13.	Larger carburetor (850 cfm instead of 650)

14.	Add a hood scoop

15.	Increase to 200 horsepower of nitrous oxide (up from 150 hp)

16.	"Experienced" driver (instead of "Good" driver)

Once again, these changes have already been made for us.  Load example vehicle #4 from the vehicle database.  Notice the results of our new design:

Est. 1/4 mile ET:  10.97 at 122.5 MPH

Keep in mind that this combination, like the others, is an actual vehicle currently running today in Houston, Texas, consistently running this E.T. and MPH.  Notice that while the engine's shift point has risen to nearly 6600 RPM, it still more or less what it was when we started in the beginning with the first combination!

While this is indeed Pro Street performance in a very buildable and driveable package, keep in mind that this combination by no means constitutes world-class Pro-Street performance.

E.T.'s in this range and faster can be attained with the same 355 small block engine, with cheap production steel heads, on 9" street tires instead of slicks, relatively quiet turbo-type mufflers with only 2 1/2" exhaust, plain Super Unleaded gas with no octane booster of any kind, a shift point below 6300 RPM, in roughly the same weight car, with 4.11 gears and only a 3000 RPM stall.  Revolutionary Concepts has had considerable personal experience with such ultimate street machines!

Why is such an engine combination not included in DRAGSTRIP PLUS' database?  Such designs are proprietary; so you will have to design combinations like this on your own.  (This is our way of issuing a challenge to our more experienced users!)

Once you master the techniques revealed in this section, with practice and experimentation you will be able to develop combinations far exceeding the abilities of the casual DRAGSTRIP PLUS user.

TIP:	If you wish to experiment and make changes to any of the combinations in your vehicle database, first use the database's COPY function to make a copy of the original, then make your changes to the copy under a new name.

Example:	Enter the database, then press <F5> and enter the record number of the vehicle you are interested in modifying.  When asked, give the new vehicle a different (unique) name.  That's all there is to it!  You can now change any thing you like, without affecting the original combination.

Just remember to save your changes and new combination(s) by pressing <F10> to exit DRAGSTRIP PLUS once you're finished, rather than just leaving the program running or turning the computer off.

## DragStrip Plus Hot Keys

Press <F1> to get on-line HELP at any time.

Press <ENTER> to continue on to the next screen.

Press <F2> to start a new vehicle.

Press <F4> to access the Vehicle Database.

Press <F7> at the E.T. Results Screen to print the current combination in its entirety.

Press <F10> to save the Vehicle Database and exit to DRAGSTRIP PLUS' main menu.

Press <SHIFT><F1> at the E.T. Results Screen to see a list of shortcut HOT KEYS around DRAGSTRIP PLUS.

Use the following shortcut HOT KEYS as a short cut to ease your way around DRAGSTRIP PLUS:

Press <CTRL><A> to jump to Advice screen

Press <CTRL><D> to jump to Vehicle Database

Press <CTRL><R> to jump to E.T. Results screen

Press <CTRL><S> to jump to Engine Statistics screen

Press <CTRL><V> to jump to Vehicle Data screen

Press <CTRL><E> to jump to Engine Data screen

# Chapter Three:  Using An Analysis Mode

## Choosing The Correct Analysis Mode

Now that you have familiarized yourself with DRAGSTRIP PLUS by working with some its sample combinations (see previous section), it's time to let DRAGSTRIP PLUS analyze your own combination.

If you do not already have DRAGSTRIP PLUS running, start the program (see prior sections in this manual for help if necessary), and enter your password.  You will then be at the "Mode Choice Selection" screen.

If you already have DRAGSTRIP PLUS loaded and do not see "Mode Choice Selection" at the top of your screen, press <F10> to exit to the main menu, then re-enter your password.  You will then be at the "Mode Choice Selection" screen.  (NOTE: you can press <F2> to take a quick shortcut to the "Mode Choice Select" screen if the words "F2=NEW VEHICLE" appear at the bottom of your screen).

Before DRAGSTRIP PLUS can begin to analyze your combination, you need to decide which analysis mode you wish DRAGSTRIP PLUS to use.  The three modes are the "Quick Compute", "Detailed Analysis", and "Dyno Approach" combination analysis modes.

Use the following descriptions of each mode to make the correct choice, based upon what you do (or don't!) know about your combination and what you expect from DRAGSTRIP PLUS:

## Quick Compute Mode

To use this mode, you must know your vehicle's accurate finish-line MPH or RPM from track experience.  In this mode, DRAGSTRIP PLUS will only calculate your vehicle's E.T., optimum gear ratio, optimum stall point, and horsepower.  You will not receive an engine analysis.

The advantages of this mode are its speed due to the very minimal amount of data you must supply, and its accuracy.

At any time you can change a "Quick Compute" analysis into a "Detailed Analysis" by selecting the "Computer Engine Analysis" option at the "VEHICLE DATA SUMMARY" screen following your vehicle's E.T.  results display.  This technique is explained in greater detail in Chapter Four further on in this manual..

SELECT YOUR CHOICE OF PROGRAM ANALYSIS MODES:

1) `Quick Compute'	You supply only parts related to the vehicle plus

its finish-line MPH or RPM, and DragStrip Plus

will compute its E.T.  Quick but highly accurate!

2) `Detailed Analysis'	You supply all engine and vehicle parts, and

DragStrip Plus will compute everything: the

vehicle's finish-line MPH, RPM, and E.T., plus

provide a comprehensive engine analysis.

3) `Dyno Approach'	You supply the vehicle's gross or net horsepower,

and DragStrip Plus will compute its E.T. and MPH.

F1=HELP       F3=SOUND<ON>        F4=DATABASE        F8=ET SCREEN        F10=END

## Detailed Analysis Mode

Most users of DRAGSTRIP PLUS use the Detailed Analysis Mode mode almost exclusively.

To use the Detailed Analysis Mode, you must have a detailed knowledge of your combination, including all major engine and vehicle parts.

This mode's main advantage is that DRAGSTRIP PLUS will compute everything related to the vehicle's performance: its finish-line MPH, RPM, E.T., and horsepower, plus provide a comprehensive vehicle and engine analysis.

You can also, if you so wish, change at any time from this mode to the much simpler "Quick Compute" analysis mode by selecting the "Ignore Engine Data" option at the "Vehicle Data Summary" screen following your vehicle's E.T. and engine analysis displays.

## Dyno Approach Mode

In this mode you supply the vehicle's gross or net horsepower, and DRAGSTRIP PLUS will compute its E.T. and finish-line MPH based upon the data you supply.

The advantages of this mode are its speed due to the minimal amount of data you must provide, and its accuracy (if the horsepower figure you provide is net horsepower).

DRAGSTRIP PLUS will NOT analyze your engine combination in this mode.  However, at any time you can switch to the "Detailed Analysis" mode by selecting the "Computer Engine Analysis" option at the "VEHICLE DATA SUMMARY" screen following your E.T.  results display.

This mode is documented in greater detail in Chapter Ten.

# Chapter Four:  Using Quick Analysis

## The Quick Compute Mode

Your vehicle's analysis in the "Quick Compute" mode starts with the  "RPM/MPH CALCULATOR" calculator.  To use this simple calculator, you must accurately know either:

1.	Your vehicle's 1/4 mile finish-line MPH

-- or --

2.	Your vehicle's 1/4 mile finish-line RPM.

If you do not know your vehicle's finish-line MPH or RPM, press <F8> to return to the opening "Analysis Mode Choice" screen, then select the "Detailed Analysis" mode to compute your vehicle.

Basically, your aim here is to use this calculator to balance your vehicle's MPH, RPM, and transmission/converter slippage, until you are satisfied with the results. Then press <F2> to continue with your vehicle's analysis.

The MPH/RPM Calculator screen looks like this:

MPH	GEAR	Tire	RPM	%Slip

...	....	..	....	10

These categories are defined as follows:

MPH	-Your vehicle's average MPH at the 1/4 mile finish-line, example "120"

GEAR	-Your gear ratio, such as 4.11 (enter as "411")

Tire	-Tire Diameter, or tire height, example "28"

RPM	-Finish line RPM at full throttle, such as "6500"

Slip	-Transmission/converter slippage, pre-assumed to be 10%.  If you have 	a manual transmission, enter "00" (for 0%).

To use the calculator, you need enter only any three out of the first four fields, then press <ENTER> to see the result.

To skip any ONE field out of the first four categories, press the <SPACE> bar.  Notice the tone change, signaling that you have jumped over this field to the following one (although no cursor is visible).

Here's a few examples of how this works:

EXAMPLE #1:	Suppose you have a vehicle that runs 124 MPH in the quarter at the track, has 4.11 gears, and 28 inch tall tires --but you are not sure what its finish-line RPM is.  In this situation, you would enter this information exactly as shown in quotes below:

"12441128" [press <ENTER>]

Notice that you must input the data as a continuous string.  The computer will neatly space it out for you.  Also notice that you do not need to enter the period (<.>) in the gear ratio; the computer automatically assumes this, for faster data entry.

EXAMPLE #2:	Suppose that you know your MPH, Gear, and RPM, but not your tire height. In this example, you would  enter the following:

"120411" [press the <SPACE> ] "6500" [press <ENTER>]

EXAMPLE #3:	Suppose that you know your car has 28 inch tall tires and 4.11 gears, and you're fairly sure your vehicle's finish-line RPM is 6500, but you don't know its quarter mile MPH. On top of that, your car has a nearly stock converter AND a tight, newly built, transmission, so you are sure your transmission slippage will be less than 10%. You estimate it to be 8%.

The data string for this car would be:

[press the <SPACE>] "4112865008" [press <ENTER>]

NOTE:	If you make a mistake entering a number, use the backspace key, or else press the <ESC> key to start over from the beginning.

IMPORTANT: Press <F2>  once done, to begin computing the vehicle's E.T. (when this option is valid, it will be lit up in green).

Press <F3>  to toggle sound ON / OFF

Press <F4>  at any time to access the program vehicle database.

Press <F8>  to return to the Analysis Mode Choice screen

Press <F10> at any time to quit the program and exit back to the DOS prompt. The vehicle database will be saved (if it has been modified) before giving you a final choice of exiting.

NOTE:	As soon as you press <F2>, DRAGSTRIP PLUS will begin asking you a series of questions, beginning with Driver's Weight.

# Chapter Five:  Entering Your Combination

## Analysis Mode

If you are using the Dyno Approach Mode, DRAGSTRIP PLUS will begin asking a series of questions, starting with Engine Horsepower.  Since relatively few DRAGSTRIP PLUS users use this mode, for further Dyno Approach Mode help, please refer to Chapter Ten, "Using Dyno Mode".

If you are using the Quick Compute or Detailed Analysis Mode, DRAGSTRIP PLUS will begin asking a series of questions, starting with Driver's Weight..  These questions are documented here in the same order as asked by the program.

## Driver's Weight

Input your weight only.  Your vehicle's weight will be inputted separately.  Separating the driver's weight from the vehicle's is useful in certain situations where there are substantial differences in weight between two drivers of the same vehicle.

In general, the less the vehicle weighs, the more noticeable the effect of changing drivers will be.

## Vehicle's Weight

Input the weight of the vehicle only.  The weight of the vehicle's driver is entered separately.  Separating the vehicle's weight from the driver's allows the program to more accurately calculate the engine's true net horsepower.

See Appendix C for commonly used vehicle weights.

## Weight Change

You can choose between having the program re-calculate your vehicle's net horsepower, or estimating the MPH, RPM, traction, and E.T. changes the weight change will have upon the vehicle.

## Nitrous Oxide Use

The program is not asking if a nitrous oxide system is on your vehicle, but whether or not a nitrous system, if any, was used to make this particular pass.

## Amount Of Nitrous Oxide Horsepower

Enter the estimated horsepower output of your Nitrous Oxide kit.  The program will assume that the Nitrous system was used in the optimum manner, i.e., throughout the quarter mile, including right off the starting line.

If you used the nitrous system for only half a pass, the program's E.T. estimates may be off by as much as a second.  This is particularly true when a nitrous system is used during only the last half of the quarter mile.

To compensate for this effect, it is acceptable to change the amount of horsepower you enter into the program.  For instance, if your vehicle has a 250 horsepower nitrous system, but was only used during the last half of the track, input 100 horse-power instead of the normal 250.

If the same 250 horsepower system is only used during the first half of the quarter mile, input 200 horsepower instead of 250. The reason for these differing horsepower input recommendations is based on the fact that nitrous has a much greater effect on a vehicle's performance when it is used during the first half of the quarter mile, verses the same system used during only the last half.

If you are using the Quick Compute Mode, it is important to realize that if you are entering your vehicle for the first time, the amount of nitrous oxide you select will have little effect upon your vehicle's estimated E.T.

This is because the program assumes that the MPH and RPM at the end of the quarter mile, as entered previously, was achieved as a result of the nitrous horsepower level you are now about to input.

Of course DRAGSTRIP PLUS can calculate what effect various amounts of nitrous oxide horsepower will have upon any given vehicle. These kind of changes should be made after the program has already calculated your vehicle's base E.T., at the data change screen. This will force MPH, RPM, gear, stall, and traction recalculations that will result in a new estimated likely E.T.

## Stall Point

IF YOU HAVE AN AUTOMATIC TRANSMISSION, the response is easy: Your stall point is the highest RPM your engine can hold in low gear with the brakes held as tightly as possible (or transbrake set) without your drive wheels beginning to spin, or the vehicle creeping forward against the brakes.

However, in certain situations, you will need to adjust this value upwards by a couple hundred or more RPM, such as if your vehicle is equipped with a poor set of tires relative to the strength of the engine, or if your vehicle has a poor set of brakes. In these situations the drive wheels will begin to spin or the vehicle creep forwards well below your transmission converter's true stall point, resulting in the need to adjust the observed stall RPM upwards by 10% or so.

IF YOU HAVE A MANUAL TRANSMISSION, only a great deal of racing experience with both manual and automatic transmissions can allow one to properly supply the correct stall point. The true stall point the program is looking for is not the RPM the clutch is `dumped' at, but is nearer the low RPM the engine is pulled down to as the clutch grabs and the vehicle gets underway.

The best approach for coming up with the correct stall point for a manual transmission is to let the program tell you what it is!  Enter your best guess, then later come back after the E.T. results screen and fudge the stall point one way or another by 15% or so, until the vehicle's E.T. looks right.

If you use this approach, be sure to watch the amount of stall loss on the E.T. Results Screen.  You can use this information to determine which way (and how much) to move your stall point guess.

## Exhaust System

Select your vehicle exhaust system according to the following definitions:

Uncapped Exhaust -refers to vehicles running open headers.

This does NOT mean open exhaust manifolds, if this is the case, you should input "Free Flowing" or "Average" in the menu selection instead of "Uncapped Exhaust".

Free Flowing	-refers to brands of exhaust mufflers that are particularly free flowing (such as Flowmaster and Dynomax) AND matching exhaust system of headers and large (3'' diameter or larger) exhaust pipes.  If you have Free Flowing mufflers but only 2 1/2'' exhaust pipes all the way through or no headers on the engine, input "Average" in the menu selection instead of "Free Flowing".

Average	-means an exhaust system with relatively small-tube headers, dual exhaust, and moderate flow mufflers such as Sonic Turbos, Hookers, etc.

Dual	-means an exhaust system that consists of exhaust manifolds, two exhaust pipes, mufflers, and no headers.

Single	-refers to an exhaust system that is comprised of exhaust manifolds, a single exhaust pipe, a muffler and/or catalytic converter, and no headers.

## Tire Height

Enter the diameter, or height of your vehicle's drive wheels in inches. DRAGSTRIP PLUS does not automatically add an allowance for tire growth, so you must manually allow for this yourself.

The amount tires "grow" varies with the tire type and other factors, such as the height and width of the tire, the speed of the vehicle, tire air pressure, and temperature of the track and tires. For example, a hard radial tire on a 90 mph in-the-quarter sports car may grow less than 1/2 inch, while a soft slick on a 290 mph Top Fuel Dragster may grow 4 inches or more.

Enter a tire diameter such as "28 1/2" as "28.5".

## Tire Selection

The type of tires on your vehicle has a lot to do with how well it "hooks up" and accelerates off the starting line.  Select the tire category that best fits the brand and type of tires you have on your vehicle according to the following definitions:

"Slicks"	-are the wide, bald, sticky racing tires used almost exclusively by the faster drag cars. Slicks are illegal for use on the street due  to the fact they float in the rain, and are generally used only at the track, or by the faster Pro Street machines.

"McCrearys"	-if you have any similar type of super traction soft-compound street racing tires such as Mickey Thompson's, etc.  These kind of tires usually wear out in just 2,000 miles or less of street use.

"Pro-Tracs"	-generally cost less than the soft-compound class of tires such As McCrearys above, and usually last a whole lot longer as well.  This type of tire usually wears out in roughly 10,000 miles of street use, and are the tires of choice for most 13-and-14 second hot rods.

"Radials"	-generally last 25,000 miles or more.  They are the tire of choice for almost all production sport cars, and most 15-and-16 second hot rods.

## Traction

The computer uses this information, in combination with other factors such as your vehicle's type of tires and its net horsepower and weight, to decide just how much E.T. your particular vehicle will lose due to traction difficulties. So think of your input as only a rough guide to the computer.

If you are using DRAGSTRIP PLUS' "Quick Compute" mode, select the traction category that best describes your vehicle's actual track performance results during the particular pass you are entering (and NOT your vehicle's normal or best traction conditions!).

If you are using DRAGSTRIP PLUS' "Detailed Analysis" or "Dyno Approach" modes, select the traction category that best describes your vehicle's BETTER track performance results (if known). If your vehicle is in the design or construction stage and has not yet made its first pass, you will need to supply DRAGSTRIP PLUS with an advance projection of your vehicle's traction.

You can arrive at this advance projection by comparing your combination with the current performances of similarly built vehicles, or if no such guides exist for your combination or are unavailable in your area, by making a careful estimation based upon logical consideration of all the facts. If you make a serious mistake, DRAGSTRIP PLUS will warn you via the "Advice Screen".

Use the following information as a guide as necessary in arriving at a projection of your vehicle's traction:

1.	ALL-OUT RACE CARS AND "TRAILER CARS" normally use slicks. These vehicles and can usually expect "Excellent" or "Good" traction; if not, such vehicles will normally be altered until they do get excellent or great traction.

2.	PRO-STREET MACHINES often employ either slicks or soft compound street tires such as McCrearys, depending upon the occasion and driver.  Such machines, if running on engine power alone, can most often expect:

"Excellent"	traction if on slicks, or "Excellent" to "Good" traction if using soft compound street tires.

On nitrous oxide, these machines can expect "Excellent" to "Satisfactory" traction if using slicks, and anywhere from "Good" to "Marginal" traction if using soft compound street tires. "Satisfactory" traction is the average for most well-built 11 second or faster street machines running 200 horsepower or more of nitrous oxide on street tires.

3.	SERIOUS STREET CARS in the 13 to 15 second range can expect anywhere from "Excellent" traction if using soft compound street tires, "Good" to "Satisfactory" traction if using Pro-Trac type tires, and "Satisfactory" to "Poor" traction if using hard radial tires and/or nitrous oxide.

4.	NEAR STOCK SPORTS CARS AND STREET CARS frequently use radial tires, but in the hands of a capable driver, can still expect "Excellent" to "Satisfactory" traction, due to these machine's relatively low horsepower outputs, high rear end gearing, and relatively heavy vehicle weights.

## DragStrip Plus' Traction Definitions

"Excellent"	traction means just that practically no perceivable tire spin on launch AT FULL THROTTLE (and "spraying right out the gates" on Nitrous Oxide, if this happens to be a NOS pass that you are entering).

"Good"	traction means either your tires quickly "hook up" off the line at full throttle (and Nitrous, if this is a NOS pass), or that in order to prevent the tires from starting to spin, you must initially leave the line at part throttle, then can immediately "floor it" (or begin to "spray") with no traction loss at all at that point right off the line.

"Satisfactory"	means that your vehicle is capable of spinning all the way down the 60 feet launching pad, except that in order to prevent this from occurring as far as possible, you have to run at half throttle (and delay use of Nitrous Oxide if this is a Nitrous pass) until near the top of first gear or the beginning of second gear.

"Marginal"	means the vehicle will "break out" and lose traction at virtually any point during the first portion of the track, if the driver were to floor it all the way (or "squeeze it" with Nitrous Oxide, if this is a Nitrous pass you are entered).

"Poor"	traction means that despite the best efforts of the driver to control it, the vehicle is liable to spin at virtually any speed or position on the track. In first gear (and often second), the vehicle is capable of smoking the tires, even at half throttle.  An good example of "Poor" traction would be would be a 12 or 11-second car attempting to use hard radial tires!

## Name Your Vehicle

If you are using the Detailed Analysis Mode, this section and the following two sections do not apply to you. You can skip the next three sections and jump directly to "Header Tubes" up ahead.

If you are using the Quick Compute Mode or Dyno Approach Mode, at this point DRAGSTRIP PLUS is going to ask you to give your vehicle a name.

If you don't give your combination some kind of a name at this point, DRAGSTRIP PLUS won't save it when you exit the program or even allow you to return to it if you load a different vehicle from the vehicle database.  Besides, any printouts you make will look better if you give your vehicle a name.

Here are a few typical examples:

"Rick's '69 street Camaro"

"Denver SideKick 355 Nova"

"454 Blue Nova Terror"

"Jeff's Dodge Dart 351"

"355 AMC Gremlin NOS Bomb"

## The E.T. Results Screen

At this point, if you are using the Quick Compute Mode and Dyno Approach Modes, DRAGSTRIP PLUS will compute and display your vehicle's E.T., MPH, RPM, Horsepower, Optimum Gear and Stall, and Biggest E.T. losses, among other things.

If DRAGSTRIP PLUS finds something critically wrong with your combination, such as excessive gear, you will be given an opportunity to change the offending entry or entries.

COMPUTER ENGINE ANALYSIS RESULTS

Vehicle: #16  Small block Nova	Date: 05-03-1994  16:07:19  3.00

MPH	Gear	Tire	RPM	Slip%

110.9	4.88	29.0	6903	10.0

Best possible ET, uncapped:	11.64	Using optimum Gear would

lower your estimated

Est	60 foot ET:	(  35.9 mph)	1.75	1/4 mile E.T. to:	11.91  sec.

Est	1/8 mile ET:	(  90.5 mph)	7.59

Est	1/4 mile ET:	(110.0 mph)	12.08	Optimum stall:	4518

Optimum 1/4 mile Gear:	5.17

Biggest E.T. losses (in order):	or  use tire height of:	27.3"

Gear	- 0.17	Optimum 1/8 mile Gear:	6.34

Stall	- 0.15	or  use tire height of:	22.3"

Traction	- 0.13

------------------	Gross Horsepower(aprox):	525.00

Total Losses: 0.45 seconds	Net Horsepower (exact):	389.23

All  calculations based upon current Elevation of 34 feet.

<F1=HELP> <F2=NEW VEHICLE> <F4=DATABASE> <ENTER=CONTINUE> <F7=PRINT> <F10=QUIT>

Press <ENTER> to continue on to the next screen when ready.

## Making Changes To Your Vehicle Combination

If you are using the Quick Compute Mode and Dyno Approach Modes, DRAGSTRIP PLUS will display a list of your vehicle's component parts and other statistics ("Change Vehicle Data" screen).

VEHICLE DATA SUMMARY

Vehicle: #16  Small block Nova	Date: 05-03-1994

MPH	Gear	Tire	RPM	Slip%

110.9	4.88	29.0	6903	10.0

1) Finish-line MPH:	110.9	G) Body Style:	Sedan

2) Gear:	4.88	H) Frontal Area:	26

3) Tire Height:	29	J) Elevation:	34

4) Finish-line RPM:	6903	K) Air Temperature:	84

5) Converter Slip%:	10	L) Air Pressure:	29.90

6) E.T.:	12.08	M) Relative Humidity:	83

7) Driver Weight:	145	N) Engine Temperature:	Warm

8) Vehicle Weight:	3400	P) Change Vehicle Name

9) Net HP:	389.23	Q) Vehicle DataBase

A) Nitrous HP:	None	R) Ignore engine data (if any)

B) Exhaust System:	Open	S) Computer engine analysis

C) Shift Point:	7318

D) Stall:	4000

E) Tires:	Slicks

F) Traction:	Good

<F1=HELP>  <F2=NEW VEHICLE>  <F4=DATABASE>  <ENTER=CONTINUE>  <F8=BACK>  <F10=QUIT>

To change any vehicle statistic, merely select its highlighted character.  DRAGSTRIP PLUS will allow you to change the value of the item you selected, then recalculate your vehicle's E.T.  and re-evaluate its advice in the light of the change you made.

At any time, if you decide to go further with your combination building and want DRAGSTRIP PLUS to analyze your engine as well, select the "Computer Engine Analysis" option.

This will convert your Quick Compute or Dyno Approach Mode analysis into the far more powerful Detailed Analysis Mode.  DRAGSTRIP PLUS will begin asking a lengthy series of engine related questions (consult this manual for Help).

If you do not know the proper responses to these questions, or change your mind about getting a Detailed Analysis, press (and hold down) the <F8> at any time to successively back up and return to this screen.

Press <ENTER> to return to E.T. Results screen.

## Header Tubes

If you are using the Quick Compute Mode or Dyno Approach Mode, the following sections do not apply to you.

If you are using the Detailed Analysis Mode, DRAGSTRIP PLUS will ask you if your engine has headers.  (NOTE: In the Detailed Analysis Mode, this question immediately follows "Traction").  DRAGSTRIP PLUS will then continue asking you other engine related questions.

If your engine combination has headers, reply yes.  You will next be asked to enter the size of the headers.

## Header Tube Size

Header tube size often has a dramatic effect upon an engine combination's horsepower output, RPM powerband, vehicle driveability, and fuel economy.

Header tubes help boost an engine's horsepower output in two ways:  by relieving exhaust back pressure, and by creating a powerful "siphon" effect which draws additional air-fuel mixture into the cylinders during the valve overlap period of the camshaft.

Generally, the larger your header tubes are, the lower the exhaust back pressure will be.  However, as header tube size increases, the cylinder siphoning effect decreases at the lower RPM ranges.  The net effect of these tradeoffs are that as header tube size increases, the powerband range of the engine increases and typically narrows.

Thus, many street cars actually run faster using a set of headers that is one size smaller than that required for peak horsepower output.  Despite the reduced horsepower, the lower engine powerband matches up better with the less-than-optimum gears and stalls used in these vehicles.  TYPICALLY, SUCH HEADERS DRAW A 88-94 RATING BY DRAGSTRIP PLUS, WHICH SHOULD BE CONSIDERED NORMAL FOR MOST STREET MACHINES.  All-out race cars generally base their combinations around header tubes which draw a 95-105 rating by DRAGSTRIP PLUS.  A popular header tube diameter for street use is 1 5/8" for small block engines, and 1 3/4" or 1 7/8" for big blocks.

## Exhaust Tube Size

The larger your exhaust pipes are, the better your engine can breathe.  Common exhaust pipe diameters for street use are 2 1/2" (typically used with "turbo" type mufflers) and 3" (typically used with free flowing type mufflers).

## Engine Size

Enter the size of your engine in cubic inch displacement (CID).  Be sure to include any extra displacement created by overboring the engine.

As engine size increases, the RPM range of the engine proportionately decreases, for any given engine combination.  Therefore, as engine size increases, your combination must be scaled upwards to reflect the increase in aspiration required to fill the larger engine to the same level of volumetric efficiency.

Overboring the engine by .030" typically adds five (5) CID if the engine is a small block, or eight (8) CID if a big block engine.  Double these figures if the engine is overbored by .060".

NOTE:	If your engine's size is measured in metric, multiply its size in cubic liters by 61.

Example: A Camaro equipped with a "5.0" engine is actually 305 CID (5.0 X 61 = 305).

Popular engine sizes are 302 and 350 CID for small block engines, and 427 and 454 CID for big block engines.

## Number Of Engine Cylinders

Press <ENTER> to accept the default number of engine cylinders already displayed, or enter a new value if applicable.

DRAGSTRIP PLUS uses this information during its process of calculating an engine's optimum fuel injector size (if the engine is fuel injected).

## Engine Manufacturer

Select the engine manufacturer that made your engine block.

## Engine Type

Select the engine category that best describes your engine.

IMPORTANT:	If you have a Ford engine equipped with Cleveland heads, select the "Big Block" category.

## Transmission Selection

Select the transmission that best describes your vehicle.  A common transmission for Chevy based vehicles for street use is the Turbo 350.

If your transmission is made by a different manufacturer, select the Chevy transmission category that comes closest to the performance and endurance of your transmission.

## Number Of Transmission Gears

You are being asked to supply the number of gears in the vehicle's transmission.  Press <ENTER> to accept the default answer already displayed, or enter the new value of your choice.

## Transmission Gear Ratios

Like your vehicle's rear end, the transmission also contains gears that have specific ratios that vary from transmission to transmission.

Transmission gear ratios work like this:  In first gear, if your vehicle's driveshaft were to rotate    3 and 1/4 times for every once the engine turned, your transmission's first gear ratio would be described as being "3.25 : 1".

A less cumbersome way to represent this ratio would be "325", and that is what DRAGSTRIP PLUS is expecting.  Press <ENTER> to accept the default value already displayed, or enter a new value of your choice.

At the current time, DRAGSTRIP PLUS makes only indirect use of this information, and is mainly included for future use.

## Finish Line Gear

You are being asked to input the transmission gear in which your vehicle crosses the finish line.

Since most race cars finish the quarter mile in high or final (1:1) gear, this is the default answer.  Press <ENTER> to accept the default, or input a different finish line gear if applicable.

NOTE:  If DRAGSTRIP PLUS detects that your vehicle's finish line RPM will be too high or low in the finish line transmission gear that you selected, DRAGSTRIP PLUS will "shift" your vehicle upwards or downwards into a different gear as applicable.

For example, if your vehicle is equipped with an overdrive transmission, DRAGSTRIP PLUS may downshift into overdrive if your vehicle's RPM across the finish line in your transmission's high (1:1) gear would exceed your engine's optimum shift point by an excessive margin.

Similarly, if your vehicle's RPM across the finish line would be below your engine's optimum shift by a considerable amount, DRAGSTRIP PLUS will upshift into your transmission's next lowest available gear.

In either case, DRAGSTRIP PLUS will inform you of the action it took via appropriate messages prior to displaying your vehicle's E.T. (in certain cases, you may be able to set your vehicle's finish-line gear back to your original setting; however, most of the time you will not want to).

## DRAGSTRIP PLUS Camshaft Type

The type of camshaft in your engine can potentially have quite an effect upon your engine's operation.

Generally, roller cams are more efficient than solid-lift camshafts, while solid-lift camshafts are more efficient are than hydraulic cams.

Roller and solid-lift cams lift the cylinder valves higher and hold them open far longer allowing useful airflow into the cylinders than hydraulic cams, for any given amount of total camshaft duration.

Most sport cars and street hot rods use hydraulic camshafts due to their ease of maintenance, while most serious street machines generally use solid cams.  Roller cams, far more expensive overall, are mainly used by all-out race cars only.

## Total Camshaft Duration

Enter your camshaft's total, or "advertised" duration. "Total duration" refers to how long the camshaft holds open a given cylinder valve above a set reference point (usually .004" valve lift), as measured in degrees of camshaft arc.

A camshaft's total duration has a direct and dramatic effect upon your engine's operation, including horsepower, shift point, powerband width, idle characteristics, fuel economy, effective compression ratio, vacuum for vehicle accessories such as power brakes, and other less significant, but important factors.  The bigger an engine is, the less effect a given camshaft will have on the engine's idle characteristics, fuel economy, and overall driveability..

DRAGSTRIP PLUS takes all these factors into consideration, therefore it is important that you enter the correct duration. If available, refer to the factory spec card included with the cam.

However, due to the algorithm employed by the program, DRAGSTRIP PLUS can tolerate a substantial degree of error, as long as you correctly answer the majority of everything else the program asks you.

See Chapter 8, "Combination Advice", for additional help in choosing the correct camshaft profile for your engine combination.

If you have a SPLIT PROFILE camshaft with different intake and exhaust durations, enter the mean (average) value of the two durations (upcoming releases of DRAGSTRIP PLUS will make this task unnecessary).  For example, if the intake valve's total duration is 280°, and the exhaust valve's total duration is 288°, enter the average value of the two figures, which is "284".

EXAMPLE:	Intake valve's  duration is .280° Exhaust valve's duration is .288°

(280 + 288) / 2 = 284 duration [average value]

A popular total duration for many mild street machines is 280 degrees;  all-out street machines employ total durations in the 290° to 325° range, depending on engine size.

See Chapter 8, "Combination Advice", for additional help in choosing the correct camshaft profile for your engine combination.

## Camshaft Duration At .050"

Enter your camshaft's duration at .050" valve lift.  This figure is determined by how long the camshaft holds open a given cylinder valve above a set reference point, .050" valve lift, as measured in degrees of camshaft arc.

The relationship between this figure, and the camshaft's total or advertised duration, taken into consideration with the maximum valve lift (both entered separately), allows DRAGSTRIP PLUS to calculate the curve of the camshaft's airflow envelope.

A camshaft's airflow envelope controls your engine's operation in virtually every aspect of its operation, including its horsepower, shift point, powerband width, idle characteristics, fuel economy, effective compression ratio, vacuum for vehicle accessories such as power brakes, and other less significant but important factors.  The bigger an engine is, the less effect a given camshaft will have on the engine's idle characteristics, fuel economy, and overall driveability..

DRAGSTRIP PLUS takes all these factors into consideration, therefore it is important that you enter the correct duration.  If available, refer to the factory spec card included with the cam.

However, due to the algorithm employed by the program, DRAGSTRIP PLUS can tolerate a substantial degree of error, as long as you correctly answer the majority of everything else the program asks you.

If you have a SPLIT PROFILE camshaft with different intake and exhaust durations at .050", enter the mean (average) value of the two durations (upcoming releases of DRAGSTRIP PLUS will make this task unnecessary).  For example, if the intake valve's duration at .050" is 230°, and the exhaust valve's duration at .050" is 236°, enter the average value of the two figures, which is "233".

EXAMPLE:	Intake valve's  duration is .230° - Exhaust valve's duration is .236°

(230 + 236) / 2 = 233 duration at .050" [average valve]

A popular duration at .050" for many street machines is 230 degrees;  all-out street machines use .050" durations in the 240° to 265° range, depending on engine size.

## Valve Lift

In general, the higher your camshaft lifts the cylinder valves, the more horsepower your engine will make, for any given camshaft duration and valve size.

If you have a SPLIT PROFILE camshaft with different intake and exhaust valve lifts, enter the mean (average) value of the two (upcoming releases of DRAGSTRIP PLUS will make this task unnecessary).  If the intake valve's lift is .480", and the exhaust valve's lift is .492", enter the average value of the two figures, which is  "486".

EXAMPLE:	Intake valve's  lift is .480" - Exhaust valve's lift is .492"

(480 + 492) / 2 = 486 lift [average value]

TECHNICAL NOTE:	Enter the valve lift as measured at the valve with the current rocker arms installed on your engine.  If you have 1.6 rocker arms but your camshaft's valve lift is rated at a 1.5 ratio (for example), and you do not know what the 1.6 rocker arm valve lift is, do the following: enter the 1.5 rocker arm valve lift figure that you are sure about, then enter 1.5 as your engine's rocker arm ratio as well (if this is a new engine combination, you will asked this question automatically later on).  Then, after DRAGSTRIP PLUS has made an initial computation of your vehicle's E.T., change the rocker arm ratio to its correct value.  DRAGSTRIP PLUS will automatically adjust the camshaft valve lift to the proper value for this new rocker arm ratio. A common valve lift for many street cars is .480"  (enter as simply "480").

## Camshaft Lobe Centers

The "camshaft lobe center separation" statistic refers to the built-in degree of physical separation between the intake and exhaust valve lobes of the camshaft, as measured in degrees of camshaft arc.

Generally, a lower number raises an engine's powerband, while a higher number lowers it.

Erson brand cams for street use are generally manufactured at 108 degrees of lobe center separation, while Competition Cams are usually set at 110 degrees.  Many stock factory camshafts use lobe centers in the range of 112 to 118 degrees., while all-out race cars frequently use cams with lobe center separations in the 106 degree range.

Be sure to enter the value shown on your camshaft card, regardless of whether your camshaft was "degreeded" at installation or not.  Later on you will be given the opportunity to enter the number of degrees your camshaft was advanced or retarded, if any.

## Camshaft Overlap

Camshaft overlap is the term used to describe the period when both of your engine's valves are open at the same time, as measured in degrees of camshaft arc.

The greater the camshaft's overlap, is, the more horsepower (potentially) the engine will make, but at an ever higher and higher RPM range.  In addition, high overlaps reduce the streetability of the vehicle, and usually waste large amounts of fuel.  However, keep in mind that the bigger your engine is, the less effect a given amount of overlap will have on the engine's idle characteristics, fuel economy, and overall driveability..

Most street cars employ camshafts in the range of 30 to 70 degrees of valve overlap, while all-out race cars can have well in excess of 100 degrees of overlap.

If your camshaft has an overlap of 60 degrees, enter this as "060", or enter as "60" and press <ENTER>.  IF YOU DO NOT KNOW YOUR CAMSHAFT'S OVERLAP, OR ARE NOT SURE, ENTER "0" (OR PRESS THE <ENTER> KEY).

However, keep in mind that DRAGSTRIP PLUS' calculation of your cam's "Overall Camshaft Profile" rating in the Advice Screen will be the most accurate if you enter your camshaft's correct overlap.

## Cam Installation Timing

A camshaft can be physically installed during engine assembly "ahead" or "behind" in its rotation, when measured in respect to the engine's crankshaft rotation, or simply installed "straight up", which is the most common assembly procedure.

In general, retarding a camshaft raises an engine's powerband peak, while advancing a cam lowers it.  The effect upon a vehicle's overall performance depends largely upon how well the various engine and vehicle parts match with each other as a total combination.

An engine which is relatively overbuilt for the vehicle usually benefits from advancing the camshaft, while an underbuilt engine normally benefits if the camshaft is retarded.  If the engine combination is balanced and matches the vehicle fairly well, the camshaft is best installed straight up.

Tip:	Most street cars benefit from advancing the camshaft.  Consult DRAGSTRIP PLUS' "Advice" screen for specific advice based upon your particular engine and vehicle combination.

A camshaft can be installed during engine assembly physically "ahead" or "behind" in its rotation when measured in respect to the engine's crankshaft rotation.

Generally, as an engine's camshaft is advanced, the engine's powerband range lowers; as it is retarded, it increases.

Enter the amount of the camshaft's deviation from "straight up" installation, in degrees of crankshaft rotation.

A popular amount to advance or retard a camshaft is 4 degrees.

## Intake Valve Size

In general, the larger your intake valves are, the better your engine will be able to breathe, and the more horsepower it will generate, for any given camshaft duration and valve lift.

Intake valve size is not as critical in supercharged and turbocharged engines, as the air-fuel mixture does not have to be drawn in to the engine, but instead is forced in under pressure.

Popular intake valve sizes for street use are 1.94", 2.02", and 2.05", entered as "194", "202", and "205" respectively.

## Exhaust Valve Size

In general, the larger your exhaust valves are, the better your engine will be able to breathe, and the more horsepower it will generate, for any given camshaft duration and valve lift.

Exhaust valve size is potentially critical in Supercharged and turbocharged engines, because while a large volume of air-fuel mixture is forced into the engine under pressure by the turbo or supercharger, the spent exhaust gases are left to basically find their own way out of the engine.

Popular exhaust valve sizes for street use are .150" and .160", or merely "150" and "160". Enter as "150" and "160" respectively.

## Rocker Arm Ratio

The greater the rocker arm ratio, the further the camshaft will open the valves for any given camshaft duration.  In addition, the greater the rocker arm ratio, the longer the camshaft will hold open the valves above any given valve lift.

Higher rocker arm ratios allow use of a smaller CAM than would otherwise be necessary to achieve a given level of performance.  Normally, a Camshaft equipped with high ratio rocker arms will produce the same horsepower using somewhat less fuel and be slightly more "driveable" than a larger Camshaft equipped with standard ratio rocker arms.

Unless your camshaft is already too large for your engine, using higher ratio rocker arms will benefit your engine.

NOTE:	If you change your engine's rocker arm ratio, DRAGSTRIP PLUS will change your camshaft's valve lift to match.  You will then be given a chance to either accept or change the new camshaft valve lift computed by DRAGSTRIP PLUS.

Small-block engines normally use rocker arm ratios of either 1.5 : 1,  1.52 : 1,  or 1.6 : 1, while big block engines most commonly use a 1.7 : 1 ratio.

A less cumbersome way to represent these ratios would be "150", "152", "160", and "170" respectively, and that is what DRAGSTRIP PLUS is expecting.

Enter a "1.52 : 1" rocker arm ratio as simply "152".

## Carburetor Type

Select the category that best describes your carburetor.  If your engine is fuel injected, select the "Injected" category.  In this event, DRAGSTRIP PLUS will assume that your fuel injection system is the optimum one for your combination.

A common carburetor for street use is the Holley.

## Carburetor Size or CFM

The larger the engine or the more radical the engine combination, the larger the carburetor must be to allow the engine to fill its cylinders to capacity.

If you use a carburetor which is moderately too small for your combination, your vehicle may run fairly close (or even the same) as its potential E.T. with the optimum carburetor, but the engine's shift point and MPH will be sharply reduced.

This is because a carburetor that is smaller (up to a certain point) than optimum will increase the engine's low-end torque, and thus aid the launch of the vehicle.  But at higher engine speeds, the engine runs out of air, forcing the driver to shift short of the engine's RPM potential.  This results in a reduction of the vehicle's top end acceleration, lowering its finish-line MPH.

Enter 750 CFM as "0750", or enter it as "750" and press <ENTER>.  A typical carburetor size for many street race cars is a carburetor with potential to flow from 600 to 850 cubic feet of airflow per minute.

## Air Induction

Select the air induction category that best fits your vehicle.  If you have a supercharger or turbocharger, you will be asked to enter the amount of boost it produces.

If your vehicle is equipped with a hood scoop that does not actually direct air into the carburetor, select either "Ram Air" or "No Hood Scoop".  The actual choice would depend upon which selection bests describes your vehicle's particular hood scoop.

## Superchargers

Superchargers are mechanically powered by a belt attached to the engine's crankshaft. Superchargers have an advantage in that they produce a constant amount of boost, regardless of engine RPM.  But they operate at the expense of fuel economy, because superchargers operate full time, whether they are really needed at the time or not.  They can also cause overheating and maintenance problems in street machines.

Superchargers are not very efficient, because of the significant engine horsepower required to power them.

NOTE:	At the present time, DRAGSTRIP PLUS makes specific use of this information only if you are using conventional racing fuels.  If you select nitromethane in combination with a supercharger, DRAGSTRIP PLUS will disregard the supercharger's actual boost completely, under the assumption you are attempting to build a Nitromethane Funny Car or Dragster.

Although DRAGSTRIP PLUS is basically not intended for use in designing vehicles in these classes, it will work on vehicles of any speed.  In this case, DRAGSTRIP PLUS assumes that you will use a blower (Supercharger) comparable in capability to those commonly used by others in the nitromethane class.  Therefore, it uses default boost values for this class of racing vehicle only.

If your combination uses conventional racing fuels, DRAGSTRIP PLUS will use the boost value you enter in its process of computing your vehicle's horse-power and E.T.

Enter your supercharger's boost in pounds per square inch (PSI).

## Turbochargers

Turbochargers are powered by waste exhaust gases from the engine.  They efficiently maximize the engine's horsepower potential at medium and upper engine speeds.  But at low engine speeds, turbochargers produce little or no boost, because the volume and velocity of waste engine exhaust gases needed to efficiently power them is sharply reduced.

Enter your turbocharger's boost in pounds per square inch (PSI).

## Turbocharger / Supercharger Boost

Turbochargers and superchargers are basically air pumps that have the potential to tremendously increase an engine's horsepower output, by forcibly packing a much larger quantity of air-fuel mix into the engine's cylinders than the engine would normally be able to do by itself employing pure cylinder vacuum only.

Enter your supercharger or turbocharger's boost in pounds per square inch (PSI).

## Number of Turbochargers

If your engine is turbocharged, you will be asked to enter the number of turbochargers installed on your engine.

## Intercooler Selection

If your engine is turbocharged, you will be asked to select the type of intercooler, if any, that is installed on your vehicle.

Intercoolers are radiator-like devices that aid the performance of turbocharged vehicles by cooling the hot intake charge produced by the turbocharger, thus allowing a denser mixture to packed into the combustion chambers of the engine.  However, this is accomplished at the expense of a boost pressure drop across the intercooler.

Aftermarket racing intercoolers work better than stock intercoolers because they cool the intake charge to a greater degree, with a smaller pressure drop across the intercooler.

Select the intercooler category that best describes your vehicle.

## Manifold Selection

Select the manifold type that best fits your vehicle according these definitions:

"Tunnel Ram"	-This is the tall, high speed, ram air effect manifold built for two carburetors.

"High-rise single plane"	-Strip Dominator type manifold.

"Medium-rise single plane"	-Victor Jr. type manifold.

"Low-rise single plane"	-Torker II type manifold.

"Dual Plane/Sports Cars"	-Performer type manifold, and most production sport car manifolds.

"Stock cast iron smog"	-This is the heavy, cast iron, flat manifold that comes standard on most non-performance V8 engine production cars for fuel economy, emission control, and cost-reduction purposes.

## Ignition Selection

Most street cars use ignition systems classified in the "High Performance" category.

At the current time, DRAGSTRIP PLUS makes no use of this information other than for general advisory purposes, and is mainly included for future uses.

## Piston Selection

Normally, serious racing applications use forged pistons, due to their greater strength and durability.

At the current time, DRAGSTRIP PLUS makes no use of this information other than for general advisory purposes, and is mainly included for future use.

## Engine Compression

The higher your engine's compression is up to a certain point, the more efficient it will be. However, the fuel octane requirements of the engine go up sharply as its compression ratio increases.

Since DRAGSTRIP PLUS looks at your engine's operational or dynamic compression ratio, rather than its static or mechanical compression ratio, the exact mechanical compression ratio is unimportant.  This is why DRAGSTRIP PLUS does not include a compression ratio calculator.

Enter a compression ratio of  "12.25 : 1"  as simply "12.25".

Most street race cars use compression ratios in the range of "8:1" to "10 : 1", because these compression ratios work well with pump gas.

## Cylinder Head Selection

The choice of the correct engine cylinder head type is one of the very few selections in DRAGSTRIP PLUS that is absolutely critical that you make correctly.

Unlike most other engine and vehicle selections used by the program, DRAGSTRIP PLUS has no way to compensate for an incorrect head type selection, except in certain situations that occur more or less infrequently.

NOTE:	Most big block engines produce more horsepower using open chamber heads than they do with closed-chamber heads.  The reverse is true with small block engines.

## Cylinder Head Porting

Porting, or enlarging a cylinder head, if done correctly, is a excellent way to (potentially) gain a lot of cheap horsepower.

How much horsepower a given engine will gain if its cylinder heads are ported depends upon a number of factors, such as the horsepower output of the engine, the type of cylinder heads in use, and especially to what degree and how well the heads are ported.

## Engine Fuel

The higher your engine's compression ratio, the greater its need for increased fuel octane.

DRAGSTRIP PLUS bases its fuel octane ratings upon your engine's operational or dynamic compression ratio, rather than its static or mechanical compression ratio.

Thus two engines built with different combinations, but with the same mechanical compression ratios and using the same fuel, may still receive different fuel octane ratings by DRAGSTRIP PLUS.

Many street machines typically use a fuel octane that draws a 85 to 95 rating by DRAGSTRIP PLUS.  As long as the vehicle's ignition timing is retarded somewhat, a fuel octane rating this low will usually work.  However, retarding the timing reduces the horsepower of the engine, lowering the vehicle's overall performance, and particularly its finish-line MPH.

The vehicle cannot attain its highest degree of performance until you use the optimum fuel octane (95-105 rating), AND the engine's ignition timing is advanced to the optimum point for the higher octane but slower burning fuel.

## Fuel Pump

If your fuel delivery system (fuel tank, fuel pump, fuel line) is inadequate to supply your engine throughout its RPM range, from launch through the finish line, the E.T.s and especially the MPHs displayed by DRAGSTRIP PLUS may be substantially faster than your vehicle's actual track results.

However, DRAGSTRIP PLUS will forecast the same E.T. for your vehicle, whether it is starving for fuel or not.  DRAGSTRIP PLUS has no way of knowing to what degree your engine is short of fuel, for a number of reasons.

It is your responsibility to make sure your engine has an adequate supply of fuel throughout the quarter mile, just as it is your responsibility to make sure the engine has been properly assembled, installed, tuned, and driven.

DRAGSTRIP PLUS will give you some warning of potential fuel delivery problems in its "Advice" screen, but this warning will not affect the vehicle's predicted E.T., no matter what fuel pump selection you make.

## Engine Quality

"Superb"  	If your vehicle is a no-breakout style Stock or Super Stock class vehicle running well below the class index, select "Superb".

"Outstanding"	If your vehicle is substantially better built than most car in its class, utilizing every conceivable engine build and chassis optimization trick possible, IRRESPECTIVE OF COST AND COMPLEXITY, or a Stock or Super Stock style vehicle running at or just above its class index, then select "Outstanding".

"Excellent"	VIRTUALLY ALL WELL BUILT COMPETITIVE VEHICLES FIT THIS CATEGORY, including ALL street and Pro cars built through common construction techniques and budgets typical of average competitive vehicles in their respective classes, with the sole exception of Stock and Super Stock style vehicles.

"Satisfactory"	If your vehicle is well built, but is somewhat out of tune, or merely getting old and starting to slow, select "Satisfactory".

"Marginal",	If your vehicle is poor or hastily assembled condition, badly mistuned, suffering severe ring blowby, or is crippled by mechanical defects, select "Marginal".

"Poor"	If your engine and vehicle is in very poor drag racing condition , select this category.

## Elevation

The higher the track elevation, the less horsepower your engine will make, and the slower your vehicle will run.  The correct elevation is important to DRAGSTRIP PLUS for another reason: your vehicle's optimum gear ratio will change dramatically with changes in elevation.

Enter your track's elevation in feet above sea level.  If you believe your track's elevation is less than 100 feet, but do not know its exact elevation, enter "0" and press the <ENTER> key (or just press <ENTER> if you have not inputted an elevation previously).

If you believe your track's elevation is greater than 100 feet, but do not know its exact elevation, call your track and ask them to provide this information.

## Air Temperature

The hotter the air temperature, the less horsepower your engine will make, and the slower your vehicle will run.  This is because hotter air is thinner than cold air, resulting in a smaller intake charge being drawn in by the engine.

Enter the air temperature at the time and place your vehicle will run.  Keep in mind that your vehicle's MPH, RPM, horsepower, and DRAGSTRIP PLUS' calculation of your combination's "optimum gear ratio" will all be affected by the air temperature you enter.

## Relative Humidity

Enter the relative humidity at the time and place of your vehicle's run.  If you do not wish to enter the relative humidity, press the <ENTER> key.  DRAGSTRIP PLUS will assume a median value of 75% as a basis for its calculations (unless you have previously entered a different humidity value).

## Barometric Air Pressure

The higher the air pressure, the more horsepower your engine will make, and the faster your vehicle will run.  As the air pressure increases, the atmosphere increases in density, resulting in a larger intake charge being drawn in by the engine.

Enter the barometric air pressure at the time and place of your vehicle's run.  If you do not wish to enter the air pressure, press the <ENTER> key.  DRAGSTRIP PLUS will assume a median value of 30.00 as a basis for its calculations (unless you have previously entered a different air pressure).

Keep in mind that your vehicle's "optimum gear ratio" is affected by the air pressure you enter.

Enter air pressure values without the decimal point, i.e., enter "30.12" as "3012".

NOTE:  Enter the "adjusted" barometric air pressures, such as those given on T.V. broadcasts and newspapers, rather than the "absolute" air pressure, typically used by aviation.

In weather suitable for drag racing, adjusted barometric air pressures typically range from 29.90 to 30.50, regardless of elevation, while absolute air pressure readings range from 27.00 to 31.50, depending upon the current elevation above sea level.

## Engine Temperature

The hotter your engine is at launch, the slower your vehicle will run.  Select the engine temperature that best describes your vehicles condition at launch per the following definitions:

"Cold"	-The engine has been allowed to cool overnight, or has been completely cooled down with a water hose.

"Warm"	-The engine has air cooled for several hours; or the engine started out cold, but the driver did a lengthy series of burnouts or a couple of stages prior to launch.

"Hot"	-The engine has air cooled an hour or less between runs.

"Burning"	-Back to back runs, such as when someone makes a hard run down the track on a hot summer day, only to drive down the return road, pull right back into the staging lanes, and take off again.

NOTE: This information primarily applies to street cars.  Because all-out race cars are typically towed back after each run, they stay cooler than street cars and remain "Cold" to "Warm".

Dragsters in particular tend to remain cool, since their engines are fully exposed to the air.  If you run a dragster, choose "Cold".

Press <ENTER> to accept the default if correct, or enter a new value.

## Vehicle Body Style

Select the body style that best describes your vehicle.

The "Fastback" body style includes relatively sleek and aerodynamic cars such as the Corvette, Camaro, GT Mustang, Trans Am, Vega, and Monza.

The "Sedan" body style includes relatively large and boxy cars such as the Chevy II Nova, Malibu, Fairlane, and Monte Carlo SS.

## Body Style Frontal Area

Enter your vehicle's total frontal body area, or cross section, in square feet.

If you do not know or do not wish to enter this figure, press the <ENTER> key.  DRAGSTRIP PLUS will make an assumption of your vehicle's frontal area based upon your vehicle's particular body style.

## Driver Experience

At the current time, DRAGSTRIP PLUS makes no use of this information, other than for informative purposes at the "Advice" screen.  Your selection will have no effect on your vehicle's E.T., no matter what choice you make.

Press <ENTER> to accept the default if correct, or make a different selection.

## Give Your Vehicle A Name

If you don't give your combination some kind of a name at this point, DRAGSTRIP PLUS won't save it when you exit the program.

In fact, you will lose your combination even if you switch to a different vehicle in the vehicle database.  Besides, any printouts you make will look better if you give your vehicle a name.

Here are a few typical examples:

"Rick's '69 street Camaro"	"Jeff's Dodge Dart 351"

"Denver SideKick 355 Nova"	"355 AMC Gremlin NOS Bomb"

"454 Blue Nova Terror"

## E.T. Results Screen

At this point, assuming DRAGSTRIP PLUS doesn't find anything critically wrong with your combination, it will compute and display your vehicle's E.T., MPH, RPM, Horsepower, Optimum Gear and Stall, and Biggest E.T. losses, among other things.

If DRAGSTRIP PLUS finds something critically wrong with your combination, it will not display your vehicle's E.T., but will stop and report "One or more fatal combination errors", followed by a list of the gross combination error(s) causing the problem.  You will then be given an opportunity to change the offending entry or entries (press <ENTER> until you come to the applicable data change screen).

NOTE:	At the present time, the vast bulk of DRAGSTRIP PLUS'  E.T. logic is geared toward 1/4 mile tracks.  The program provides general ballpark 60 foot and 1/8 mile E.T. and MPH estimates for the sake of the user's curiosity only -- no accuracy is implied over these shorter distances.

COMPUTER ENGINE ANALYSIS RESULTS

Vehicle: #16  Small block Nova	 Date: 05-03-1994  13:41:29  3.00

MPH	Gear	Tire	RPM	Slip%

110.9	4.88	29.0	6903	10.0

Best possible ET, uncapped:	11.64	Using optimum Gear would

lower your estimated

Est	60 foot ET:	(  35.9 mph)	1.75	1/4 mile E.T. to:	11.91  sec.

Est	1/8 mile ET:	(  90.5 mph)	7.59

Est	1/4 mile ET:	(110.0 mph)	12.08	Optimum stall:	4518

Optimum 1/4 mile Gear:	5.17

Biggest E.T. losses (in order):	or  use tire height of:	27.3"

Gear	- 0.17	Optimum 1/8 mile Gear:	6.34

Stall	- 0.15	or  use tire height of:	22.3"

Traction	- 0.13

------------------	Gross Horsepower(aprox):	525.00

Total Losses: 0.45 seconds	Net Horsepower (exact):	389.23

All calculations based upon current Elevation of 34 feet.

<F1=HELP> <F2=NEW VEHICLE> <F4=DATABASE> <ENTER=CONTINUE> <F7=PRINT> <F10=QUIT>

NO ATTEMPT SHOULD BE MADE TO USE THE 60 FOOT, 1/8 MILE, HORSEPOWER, OR  TORQUE FIGURES TO OPTIMIZE YOUR ENGINE COMBINATION.

Instead, see the E.T. Results, Advice and Engine Statistics screens for specific computer suggestions to optimize your engine's performance.

COMBINATION PART RATINGS

Vehicle: #16  Small block Nova	Date: 05-03-1994

RATING OF 100 = PERFECT:	Computer Suggestion(s):

-------------------------------	-------------------------------------

Fuel octane	113.5	Advance CAM -or- add more gear

Head type	110.3

Cam type	109.7	Try increasing Cam lobe centers

Total Duration	105.9

Header size	101.1	If this is a street machine:

Intake Manifold	99.3	Try smaller headers. (Press F1 for Help)

Compression	97.2

Exhaust valve size	96.9	Try larger carburetor(s)

Overall Camshaft Profile	96.9

Port job	96.3	Traction may be better than "Good"

Carburetor CFM	95.9

Intake valve size	95.4	Using lower fuel octane will help (if

Cam Overlap	94.3	distributor timing on vehicle is reset)

Duration @.050"	94.0

Valve lift	93.3	Cooling engine completely would help

<F1=HELP>  <F4=DATABASE>  <F7=PRINT>  <ESC=EXIT>  <ENTER=CONTINUE>  <F10=QUIT>

# Chapter Six:  E.T. Discrepancy Errors

## E.T. Discrepancy

What to do if your vehicle's E.T. comes out wrong

Don't expect DRAGSTRIP PLUS to always nail on the first try the precise performance of the combinations you give it -- particularily if the combinations you enter are someone else's.

In fact, DRAGSTRIP PLUS will predict a vehicle's E.T. correctly only about half the time on the first try.  The rest of the time you will have to juggle the vehicle's input data to correct the predicted E.T.

Here are the biggest contributing factors to an incorrectly predicted E.T., listed in order of greatest importance and frequency of occurrence:

1.	Engine mis-tuned, mis-assembled, and/or poorly driven.  DRAGSTRIP PLUS expects your vehicle to be flawlessly assembled, perfectly tuned, and superbly driven (if you have selected "Excellent" or better engine construction quality).

2.	Insufficient fuel delivery to the engine during part or all of the run.

3.	Traction losses incorrectly described to the program (re-read traction help).

4.	Camshaft lobe centers, overlap, and/or cam installation timing unknown or incorrectly entered by user.

5.	Vehicle's actual weight differs from that entered into the program.

6.	Air temperature as entered into DRAGSTRIP PLUS differs significantly from the actual air temperature at the time of the vehicle's run.

7.	The engine's actual shift point differs substantially from the shift point predicted by the program.

While there are several contributing causes to a major shift point discrepancy, the most common cause is an engine that is seriously overbuilt for its underlying capabilities.

For an extreme example, suppose that an otherwise STOCK small block engine was outfitted with a set of massively ported aluminum heads, .800" lift roller cam, large tube headers, and a large 1150 CFM Dominator carburetor.

From an airflow capability standpoint, this combination would easily rev to 8000 RPM or higher, but yet if you actually tried to shift it at this point, the engine would explode.  The stock rods, crank, valve train, fuel delivery system, ignition system, etc., would prevent this engine from being shifted much higher than 5500 or 6000 RPM, well below this engine's true (combination) shift point of 8000 RPM.

Since DRAGSTRIP PLUS takes it for granted that you built your combination in the optimum manner (if you have selected "Excellent" or better engine construction quality), its prediction of this engine's shift point would be way too high (8000 RPM or above), resulting in a E.T.  that could easily be off by a second or even more (typically on the slow side).

Short of rebuilding such an engine from scratch, the only other solutions to this problem would be to either select a different engine construction quality such as "Poor" or "Satisfactory", and/or manually set the engine's shift point in DRAGSTRIP PLUS yourself, thus overriding the program's predicted shift point.  Better than nine times out of ten, this will result in the program forecasting an E.T. that is quite close to actuality.  To manually set an engine's shift point, press <ENTER> until you reach the "VEHICLE DATA CHANGE SCREEN", then select the shift point option.

Once again, in order to obtain the best results with DRAGSTRIP PLUS, it is important that you read and thoroughly understand all Help information provided by DRAGSTRIP PLUS for its required data inputs.  On-line Help is accessible throughout the program by pressing the <F1> key, and it should be systematically consulted until you become thoroughly familiar with this program.

If you cannot correct your vehicle's E.T. by following the steps listed above, please contact our technical department at any time and leave a description of your problem along with your name and number, and we will be happy to help you resolve it.  Our technical support number will be displayed on your screen as you exit DRAGSTRIP PLUS.

NOTE: NO ATTEMPT SHOULD BE MADE TO USE THE 60 FOOT, 1/8 MILE, HORSEPOWER, OR  TORQUE FIGURES TO OPTIMIZE YOUR ENGINE COMBINATION.

Instead, see the E.T. Results, Advice and Engine Statistics screens for specific computer suggestions to optimize your engine's performance.

# Chapter Seven:  Analysis And Ratings

## Engine Analysis And Ratings

Following is a definition of terms used on the ENGINE ANALYSIS SCREEN:

"Combination / Design Rating"

This is the computer's ranking of your engine's overall design as a combination.  The better your combination's parts work together, the higher this design rating will be.  Street machines generally have lower ratings (in the 70 to 80 range) than all-out race cars do.

"Engine-to-Vehicle Matchup Rating"

This rating tells how well your engine matches your vehicle.  A low rating (60 or below) might well indicate a gross mismatch, such as a 9 second engine in street car, or the reverse of a street engine in a 9 second car.  Street machines normally draw lower ratings (in the 70 to 85 range) than all-out race cars do.  THE HIGHER THIS RATING IS, THE FASTER YOUR VEHICLE WILL BE --EVEN IF YOUR FINAL ENGINE COMBINATION PRODUCES LESS HORSEPOWER THAN YOUR ORIGINAL COMBINATION.

"Engine Overall Power Rating"

This is a measure of how radical the computer thinks your engine combination is (irrespective of actual engine size), with "10" representing something absolutely slow and low performance like a stock Volkswagon Beetle, and "90" representing a 4 second Top Fuel Dragster.

"Computer Estimated Shift Point"

This is the computer's estimate of the shift point necessary for your combination to attain its highest possible finish-line MPH, i.e., the MPH shown at the top of the screen.  If you cannot shift at this RPM, your engine is either not built or tuned correctly.  For further information, read "Help" for the E.T. Results screen.  See also Help under the category of "shift point".

"Estimated Engine Powerband Width"

The wider your engine's powerband is, the faster your vehicle will (potentially) be.  Engines with wide powerbands need less stall and even less gear than engines with thin, narrow powerbands.

"Engine Size in CID"

Your engine's size (Cubic Inch Displacement).

"Computer Predicted HP per CID"

This rating is arrived at by dividing the computer's estimation of your engine's total gross horsepower at the flywheel with open exhaust by your engine's size (CID).  This figure is a matter of pride to many engine builders, and can be used to settle disputes as to which of two engine combinations is truly the most powerful from a relative standpoint, ignoring engine size and other combination specifics.

"Computer Est. of Lost Exhaust HP"

This is the computer's estimation of your vehicle's exhaust system losses.  This rating does not apply to vehicles with open headers, as the computer does not include header tube losses in this rating.

"Nr. of Badly Mismatched Parts"

Poorly designed or thin powerband engines often have one or more badly mismatched engine parts in their combinations.  Street machines with two or more badly mismatched parts may exhibit poor driveability.

Badly mismatched engine components are highlighted in red.

"Highest Rated Engine Part(s)"

Any engine component that is rated at "100" by the computer is perfectly matched for your combination.  Generally, any rating over 100 means the affected engine component is either "too radical", or just simply unnecessary and not needed for your combination's optimum performance.  The difference depends upon the part.  A carburetor or intake manifold that draws a 110 ranking is "too radical", whereas a Camshaft or head port job that draws the same 110 rating is simply "unnecessary" for optimum cost vs. performance considerations and / or driveability.

"Lowest Rated Engine Part(s)"

Generally, any rating under 100 means the affected engine component is "insufficient" or "not enough" for your combination's optimum performance.  However, headers tubes are the main exception to this rule.  Many street cars perform better with headers ranked below 100, typically 92 to 98.

ENGINE STATISTICS SCREEN

Vehicle: #16 Small block Nova	Date: 05-03-1994

Combination / Design Rating:	85.6	Optimum Carb CFM: 800 - 1049

Engine-to-Vehicle Matchup Rating:	86.6	Opt Primary Header Len: 34.4"

Engine Overall Power Rating:	53.7

Computer Estimated Shift Point:	7318	Shift MPH is 1st gear:	47.3

Estimated Engine Powerband Width:	2800	2nd gear:	66.2

3rd gear:	116.4

Engine Size in CID :	383

Computer Predicted HP per CID:	1.28

Computer Est. of Lost Exhaust HP:	N/A

Nr. of Badly Mismatched Parts:	NONE

Highest Rated Engine Part(s):	Fuel octane	Rating:	113.5

(100 = Perfect)	Head type	Rating:	110.3

Lowest Rated Engine Part(s):	Duration @.050"	Rating:	94.0

Valve lift	Rating:	93.3

<F1=HELP>  <F2=NEW VEHICLE>  <F3=ADVICE>  <ENTER=CONTINUE>  <F8=BACK>  <F10=QUIT>

# Chapter Eight:  Combination Advice

## DRAGSTRIP PLUS Combination Advice

If you pressed the <F3> key at the "Engine Statistics Screen", DRAGSTRIP PLUS displays the optional Advice screen (see following page).

Any engine component that receives a "100" rating by the computer should be considered  perfectly matched for your combination.

Generally, any part rated over 100 means the affected component is either "too radical", or simply just unnecessary and not needed for your combination's optimum performance.  The difference depends upon the part.  A carburetor or intake manifold that draws a 110 ranking is "too radical", whereas a camshaft or head port job that draws the same 110 rating is simply "unnecessary" for optimum cost versus performance considerations and / or street driveability.

Any part rated under 100 means the affected engine component is "insufficient" or "not enough" for your combination's optimum performance.  HOWEVER, HEADER TUBES ARE THE PRIMARY EXCEPTION TO THIS RULE.  Many street cars perform significantly better with headers ranked well below 100, typically in the 92 to 98 range, because the resulting lower engine RPM power band works better with the undersized gears and stalls most street machines typically employ.

WARNING:	Header tubes with rankings in the 88 to 95 range may exhibit chronic exhaust reversion side effects, such as continually blown or leaking header gaskets and / or continually burned or worn out cylinder head exhaust valve guides.

ANOTHER DECEPTIVELY IMPORTANT STATISTIC IS THE CARBURETOR SIZE RATING.  A carburetor rating of 96 would naturally seem to indicate that although the carburetor's size is somewhat small, it ought to be satisfactory.  In reality, a carb rated at 96 or below can have a significant negative impact on your vehicle's performance.  If your carburetor size is rated below 100, you should try larger carburetors and carefully note the resulting shift points, MPHs, and overall E.T.s.  A slightly lower rear end gear ratio may be necessary to compensate for the resulting increased (raised) engine RPM power band range.

COMBINATION PART RATINGS

Vehicle: #16  Small block Nova	Date: 05-03-1994

RATING OF 100 = PERFECT:	Computer Suggestion(s):

-------------------------------	-------------------------------------

Fuel octane	113.5	Advance CAM -or- add more gear

Head type	110.3

Cam type	109.7	Try increasing Cam lobe centers

Total Duration	105.9

Header size	101.1	If this is a street machine:

Intake Manifold	99.3	Try smaller headers. (Press F1 for Help)

Compression	97.2

Exhaust valve size	96.9	Try larger carburetor(s)

Overall Camshaft Profile	96.9

Port job	96.3	Traction may be better than "Good"

Carburetor CFM	95.9

Intake valve size	95.4	Using lower fuel octane will help (if

Cam Overlap	94.3	distributor timing on vehicle is reset)

Duration @.050"	94.0

Valve lift	93.3	Cooling engine completely would help

<F1=HELP>   <F4=DATABASE>  <F7=PRINT>   <ESC=EXIT>   <ENTER=CONTINUE>   <F10=QUIT>

Overall camshaft profile: DRAGSTRIP PLUS rates your camshaft in two ways: by its individual statistics, and by the camshaft lobe profile's overall effectiveness in matching up with your engine combination as a whole.  Your camshaft's matchup with your engine is called the "Overall Camshaft Profile" rating.

This statistic rates only the camshaft's lobe profile; it does not consider the camshaft's lobe center separation, or whether the camshaft is advanced or retarded (both of which are considered separately).

While DragStrip Plus can easily determine whether any given camshaft is too large or small for any engine combination, only you can determine whether it is the camshaft that is incorrect for the engine, or whether it is engine that is too stock or radical for the camshaft.  You must make this determination based upon the direction you wish your combination to take.

Keep in mind that even if DragStrip Plus gives your camshaft a perfect "Overall Camshaft Profile" rating of 100, this does not mean that a still better camshaft profile does not exist.  For example, a roller cam profile rated at 100 will potentially produce far more horsepower than will a hydraulic cam profile also rated at 100.

Among other reasons, this is because certain camshaft profiles (especially hydraulic cams) may have excessive spread between their total duration and duration at .050" statistics, occasionally generating a variety of messages such as "too much total duration".  Yet this same camshaft profile may still be given an overall ranking of 100, based upon other considerations.

In short, in order to determine the best camshaft for your engine combination, you should experiment with various combinations of camshaft profiles, lobe center separations, and installation timings (advance and retard each cam you try).

# Chapter Nine:  Combination Changes

## Vehicle Data Screen

To change any vehicle statistic, merely select its highlighted character.  DRAGSTRIP PLUS will allow you to change the value of the item you selected, then recalculate your vehicle's E.T. and re-evaluate all its suggestions in the light of the change you made.

Select the "Ignore Engine Data" option to force DRAGSTRIP PLUS to calculate your vehicle's E.T. based solely upon its finish-line MPH.  This works assuming you entered your combination into the program using the "Detailed Analysis" mode.  This basically turns a "Detailed Analysis" into a "Quick Compute" analysis; a reminder message will appear at the top of the E.T. Results screen.  It is easy to reverse this selection [see following].

Select the "Computer Engine Analysis" option to force DRAGSTRIP PLUS to calculate your vehicle's E.T. based upon its engine combination.  If you have not previously entered your combination's engine related data into the program, you will be asked to do so.  This selection basically turns a "Quick Compute" analysis into a "Detailed Analysis".  It also reverses the effects of the "Ignore Engine data" option (if previously selected).

VEHICLE DATA SUMMARY

Vehicle: #16  Small block Nova	Date: 05-03-1994

MPH	Gear	Tire	RPM	Slip%

110.9	4.88	29.0	6903	10.0

1) Finish-line MPH:	110.9	G) Body Style:	Sedan

2) Gear:	4.88	H) Frontal Area:	26

3) Tire Height:	29	J) Elevation:	34

4) Finish-line RPM:	6903	K) Air Temperature:	84

5) Converter Slip%:	10	L) Air Pressure:	29.90

6) E.T.:	12.08	M) Relative Humidity:	83

7) Driver Weight:	145	N) Engine Temperature:	Warm

8) Vehicle Weight:	3400	P) Change Vehicle Name

9) Net HP:	389.23	Q) Vehicle DataBase

A) Nitrous HP:	None	R) Ignore engine data (if any)

B) Exhaust System:	Open	S) Computer engine analysis

C) Shift Point:	7318

D) Stall:	4000

E) Tires:	Slicks

F) Traction:	Good

<F1=HELP>  <F2=NEW VEHICLE>  <F4=DATABASE>  <ENTER=CONTINUE>  <F8=BACK>   <F10=QUIT>

Press <ENTER> to continue on to the next screen.

## Engine Data Screen

To change any engine statistic, merely select its highlighted character.  DRAGSTRIP PLUS will allow you to change the value of the item you selected, then recalculate your vehicle's E.T. and re-evaluate all its suggestions in the light of the change you made.

ENGINE DATA SUMMARY

Vehicle: #16  Small block Nova	Date: 05-03-1994

1) Header Tube size:	1 3/4	E) Ex.Valve Size:	1.60

2) Exh. Pipe size:	N/A	F) Rocker Ratio:	1.5 : 1

3) Engine CID:	383 Chevy	G) Carburetor:	Holley

4) Transmission:	Turbo 350	H) Carb. CFM:	850

1st Gear:	2.46	J) Air Induction:	Hood Scoop

2nd Gear:	1.76	K) Boost:	N/A

3rd Gear:	1.00	L) InterCooler:	N/A

4th Gear:	N/A	M) Manifold:	Med.-rise single plane

5th Gear:	N/A	N) Ignition:	Hi-performance

5) Finish Line Gear:	3rd	P) Pistons:	Forged

6) Camshaft:	Solid	Q) Compression:	10 : 1

7) Tot. Duration:	310	R) Head Type:	Closed-Chamber iron

8) Dur. @ .050":	260	S) Port Job:	Thorough

9) Valve lift:	.570"	T) Fuel:	110+ octane Racing Fuel

A) Lobe Centers:	108	U) Fuel pump:	125+ gpm Electric

B) Cam Overlap:	75	V) Engine Quality:Excellent (Most Cars)

C) Cam Timing:	3 Advanced	W) Driver Skill:	Experienced

D) In. Valve Size:	2.02

<F1=HELP>  <F2=NEW VEHICLE>  <F4=DATABASE>  <ENTER=CONTINUE>  <F8=BACK>  <F10=QUIT>

Press <ENTER> to continue on to the next screen.

# Chapter Ten:  Using Dyno Mode Analysis

## Using The Dyno Approach Mode

When using the Dyno Approach Mode DRAGSTRIP PLUS will begin asking a series of questions, starting with engine horsepower.

NOTE:	Since relatively few users of DRAGSTRIP PLUS make use of the program's Dyno Approach mode, this chapter was moved toward the rear of this manual.

## Engine Horsepower

The first question asked by DRAGSTRIP PLUS in Dyno Approach Mode regards your engine's horsepower.

Enter your engine's horsepower (as produced at the same elevation above sea level the vehicle will actually be run).  You will next be asked whether this entry is "Gross" or "Net" horsepower.

Keep in mind that if you enter your vehicle's net (rear wheel) horsepower rather than its gross (flywheel) horsepower, the resulting E.T.s will be far more accurate.

## Gross And Net Horsepower

If the horsepower figure you entered was measured at the engine flywheel, select "Gross" horsepower.  Dynos measure horsepower this way; cars built during the 1960's were rated this way.

If the horsepower figure you entered was measured at the vehicle's rear wheels, select "Net" horsepower.  All new cars are rated in net horsepower.  In addition, some businesses such as "Brake Check", measure horsepower this way.

Press <ENTER> to accept the default, which is displayed at the top of the screen.

## Driver's Weight

Input your weight only.  Your vehicle's weight will be entered separately.  Separating the driver's weight from the vehicle's is useful in certain situations where there are substantial differences in weight between two drivers of the same vehicle.

In general, the less the vehicle weighs, the more profound the effect of changing drivers will be.

## Vehicle's Weight

Input the weight of the vehicle only.  The weight of the vehicle's driver is entered separately.  Separating the vehicle's weight from the driver's allows the program to more accurately calculate the engine's true net horsepower.

See Appendix C for commonly used vehicle weights.

## Weight Change

You can choose between having the program re-calculate your vehicle's net horsepower, or estimating the MPH, RPM, traction, and E.T. changes the weight change will have upon the vehicle.

## Nitrous Oxide Use

The program is not asking if a nitrous oxide system is on your vehicle, but whether or not a nitrous system, if any, was used to make this particular pass.

## Amount Of Nitrous Oxide Horsepower

Enter the estimated horsepower output of your Nitrous Oxide kit.  The program will assume that the Nitrous system was used in the optimum manner, i.e., throughout the quarter mile, including right off the starting line.

If you used the nitrous system for only half a pass, the program's E.T. estimates may be off by as much as a second.  This is particularly true when a nitrous system is used during only the last half of the quarter mile.

To compensate for this effect, it is acceptable to change the amount of horsepower you enter into the program.  For instance, if your vehicle has a 250 horsepower nitrous system, but was only used during the last half of the track, input 100 horse-power instead of the normal 250.

If the same 250 horsepower system is only used during the first half of the quarter mile, input 200 horsepower instead of 250.  The reason for these differing horsepower input recommendations is based on the fact that nitrous has a much greater effect on a vehicle's performance when it is used during the first half of the quarter mile, verses the same system used during only the last half.

If you are using the Quick Compute Mode, it is important to realize that if you are entering your vehicle for the first time, the amount of nitrous oxide you select will have but little effect upon your vehicle's estimated E.T.

This is because the program assumes that the MPH and RPM at the end of the quarter mile, as entered previously, was achieved as a result of the nitrous horsepower level you are now about to input.

Of course DRAGSTRIP PLUS can calculate what effect various amounts of nitrous oxide horsepower will have upon any given vehicle.  These kind of changes should be made after the program has already calculated your vehicle's base E.T., at the data change screen.  This will force MPH, RPM, gear, stall, and traction recalculations that will result in a new estimated likely E.T.

## Nitrous Horsepower Usage

If you select the first option, DRAGSTRIP PLUS WILL NOT ADD the Nitrous horsepower you just selected to your engine's overall horsepower output.  If you select the second option, DRAGSTRIP PLUS WILL ADD the (gross) Nitrous horsepower you just selected to your engine's overall horsepower output.

## Stall Point

IF YOU HAVE AN AUTOMATIC TRANSMISSION, the response is easy: Your stall point is the highest RPM your engine can hold in low gear with the brakes held as tightly as possible (or transbrake set) without your drive wheels beginning to spin, or the vehicle creeping forward against the brakes.

However, in certain situations, you will need to adjust this value upwards by a couple hundred or more RPM, such as if your vehicle is equipped with a poor set of tires relative to the strength of the engine, or if your vehicle has a poor set of brakes.  In these situations the drive wheels will begin to spin or the vehicle creep forwards well below your transmission converter's true stall point, resulting in the need to adjust the observed stall RPM upwards by 10% or so.

IF YOU HAVE A MANUAL TRANSMISSION, only a great deal of racing experience with both manual and automatic transmissions can allow one to properly supply the correct stall point.  The true stall point the program is looking for is not the RPM the clutch is "dumped" at, but is nearer the low RPM the engine is pulled down to as the clutch grabs and the vehicle gets underway.

The best approach for coming up with the correct stall point for a manual transmission is to let the program tell you what it is!  Enter your best guess, then later come back after the E.T. results screen and fudge the stall point one way or another by 15% or so until the vehicle's E.T. looks right.

If you use this approach, be sure to watch the amount of stall loss on the E.T.  results screen. You can use this information to determine which way (and how much) to move your stall point guess.

## Exhaust System

Select your vehicle exhaust system according to the following definitions:

Uncapped Exhaust -refers to vehicles running open headers.

This does NOT mean open exhaust manifolds, if this is the case, you should input "Free Flowing" or "Average" in the menu selection instead of "Uncapped Exhaust".

Free Flowing	 -refers to a brand of exhaust mufflers that are particularly free flowing (such as Flowmaster and Dynomax) AND matching exhaust system of headers and large (3'' diameter or larger) exhaust pipes.  If you have Free flowing type mufflers but have only 2 1/2'' exhaust pipes all the way through or no headers on the engine, input "Average" in the menu selection instead of "Free Flowing".

Average	-means an exhaust system with relatively small-tube headers, dual exhaust, and average mufflers such as Sonic Turbos, Hookers, etc.

Dual	-means an exhaust system that consists of exhaust manifolds, two exhaust pipes, mufflers, and no headers.

Single	-refers to an exhaust system that is comprised of exhaust manifolds, a single exhaust pipe, a muffler and/or catalytic converter, and no headers.

## Tire Height

Enter the diameter, or height of your vehicle's drive wheels in inches. DRAGSTRIP PLUS does not automatically add an allowance for tire growth, so you must manually allow for this yourself.

The amount tires "grow" varies with the type of tire and other factors, such as the height and width of the tires, the speed of the vehicle, tire air pressure, and temperature of the track and tires.  For example, a hard radial tire on a 90 mph in-the-quarter sports car may grow less than 1/2 inch, while a soft slick on a 300 mph Top Fuel Dragster may grow 4 inches or more.

Enter a tire diameter such as "28 1/2" as "28.5".

## Tire Selection

The type of tires on your vehicle has a lot to do with how well it "hooks up" and accelerates off the starting line.  Select the tire category that best fits the brand and type of tires you have on your vehicle according to the following definitions:

"Slicks"	-are the wide, bald, sticky racing tires used almost exclusively by the faster drag cars.  Slicks are illegal for use on the street due  to the fact they float in the rain, and are generally used only at the track, or by the faster Pro Street machines.

"McCrearys"	-if you have any similar type of super traction soft-compound street racing tires such as Mickey Thompson's, etc.  These kind of tires usually wear out in just 2,000 miles or less of street use.

"Pro-Tracs"	-generally cost less than the soft-compound class of tires such As McCrearys above, and usually last a whole lot longer as well. This type of tire usually wears out in roughly 10,000 miles of street use, and are the tires of choice for most 13-and-14 second hot rods.

"Radials"	-generally last 25,000 miles or more.  They are the tire of choice for almost all production sport cars, and most 15-and-16 second hot rods.

## Traction

The computer uses this information, in combination with other factors such as your vehicle's type of tires and its net horsepower and weight, to decide just how much E.T. your particular vehicle will lose due to traction difficulties.  So think of your input as only a rough guide to the computer.

If you are using DRAGSTRIP PLUS' "Quick Compute" mode, select the traction category that best describes your vehicle's actual track performance results during the particular pass you are entering (and NOT your vehicle's normal or best traction conditions!).

If you are using DRAGSTRIP PLUS' "Detailed Analysis" or "Dyno Approach" modes, select the traction category that best describes your vehicle's BETTER track performance results (if known). If your vehicle is in the design or construction stage and has not yet made its first pass, you will need to supply DRAGSTRIP PLUS with an advance projection of your vehicle's traction.

You can arrive at this advance projection by comparing your combination with the current performances of similarly built vehicles, or if no such guides exist for your combination or are unavailable in your area, by making a careful estimation based upon logical consideration of all the facts. If you make a serious mistake, DRAGSTRIP PLUS will warn you via the "Advice Screen".

Use the following information as a guide as necessary in arriving at a projection of your vehicle's traction:

1.	ALL-OUT RACE CARS AND "TRAILER CARS" normally use slicks. These vehicles and can usually expect "Excellent" or "Good" traction; if not, such vehicles will normally be altered until they do get excellent or great traction.

2.	PRO-STREET MACHINES often employ either slicks or soft compound street tires such as McCrearys, depending upon the occasion and driver.  Such machines, if running on engine power alone, can most often expect:

"Excellent"	traction if on slicks, or "Excellent" to "Good" traction if using soft compound street tires.

On nitrous oxide, these machines can expect "Excellent" to "Satisfactory" traction if using slicks, and anywhere from "Good" to "Marginal" traction if using soft compound street tires.  "Satisfactory" traction is the average for most well-built 11 second or faster street machines running 200 horsepower or more of nitrous oxide on street tires.

3.	SERIOUS STREET CARS in the 13 to 15 second range can expect anywhere from "Excellent" traction if using soft compound street tires, "Good" to "Satisfactory" traction if using Pro-Trac type tires, and "Satisfactory" to "Poor" traction if using hard radial tires and/or nitrous oxide.

4.	NEAR STOCK SPORTS CARS AND STREET CARS frequently use radial tires, but in the hands of a capable driver, can still expect "Excellent" to "Satisfactory" traction, due to these machine's relatively low horsepower outputs, high rear end gearing, and relatively heavy vehicle weights.

## DragStrip Plus' Traction Definitions

"Excellent"	traction means just that practically no perceivable tire spin on launch AT FULL THROTTLE (and "spraying right out the gates" on nitrous oxide, if this happens to be a nitrous pass that you are entering).

"Good"	traction means either your tires quickly "hook up" off the line at full throttle (and nitrous, if this is a NOS pass), or that in order to prevent the tires from starting to spin, you must initially leave the line at part throttle, then can immediately "floor it" (or begin to "spray") with no traction loss at all at that point right off the line.

"Satisfactory"	means that your vehicle is capable of spinning all the way down the 60 feet launching pad, except that in order to prevent this from occurring as far as possible, you have to run at half throttle (and delay use of nitrous oxide if this is a nitrous pass) until near the top of first gear or the beginning of second gear.

"Marginal"	means the vehicle will "break out" and lose traction at virtually any point during the first portion of the track, if the driver were to floor it all the way (or "squeeze it" with nitrous oxide, if this is a nitrous pass you are inputting).

"Poor"	traction means that despite the best efforts of the driver to control it, the vehicle is liable to spin at virtually any speed or position on the track.  In first gear (and often second), the vehicle is capable of smoking the tires, even at half throttle.  An good example of "Poor" traction would be would be a 12 or 11-second car attempting to use hard radial tires!

## Naming Your Vehicle

At this point DRAGSTRIP PLUS is going to ask you to give your vehicle a name.

If you don't give your combination some kind of a name at this point, DRAGSTRIP PLUS won't save it when you exit the program or even allow you to return to it if you load a different vehicle from the vehicle database.  Besides, any printouts you make will look better if you give your vehicle a name.

Here are a few typical examples:

"Rick's '69 street Camaro"

"Denver SideKick 355 Nova"

"454 Blue Nova Terror"

"Jeff's Dodge Dart 351"

"355 AMC Gremlin NOS Bomb"

# Appendix  A

## Changing Computer's Shift Point

If used the "Detailed Analysis" mode to compute your vehicle, your combination's optimum engine shift point will have been set by the computer.

Still, at any time you can change the computer's estimation of your vehicle's optimum shift point to whatever value you choose; however, keep in mind that your vehicle's MPH will be frozen at its current value regardless of subsequent changes to other vehicle stats, until the `Computer Engine Analysis' option is selected at the Vehicle Data Summary screen.

This will cause the computer to replace your shift point estimate with its own, and allow it to once again compute what it thinks your vehicle's finish-line MPH will be (and allow it to generate reports on your engine combination again!).

The engine shift point is one of the few stats that is fairly critical.  Note that changing the shift point has a fairly dramatic effect upon the vehicle's E.T.;  resist the temptation to experiment with this and enter only what shift point you know from track experience will result in the vehicle's highest possible finish-line MPH (note the emphasis is on track MPH, not track E.T). Any other method will result in grossly inaccurate results.

See chapter 6 ("E.T. Discrepancy Errors") for more information on this topic.

## Transmission Slippage Change

Once you enter your transmission's slippage, you can choose between having the program re-calculate your vehicle's finish-line MPH, or calculating its new finish-line RPM.

The rate your transmission falls behind your engine in its rotation is call "transmission slippage", or in case of an automatic transmission "converter slippage".

Most automatic transmission converters slip an average of 10%.  This is the default valve used by DRAGSTRIP PLUS.  If you believe converter slips somewhat less than average, enter "08" for 8%;  if you think your converter is somewhat looser than average, enter 12%.

Manual transmissions by nature exhibit virtually no slippage.  If you have a manual transmission, enter "00" for 0%.

# Appendix  B

## Unfinished Vehicle In Database

If you exit DRAGSTRIP PLUS while you are in the middle of entering a new vehicle, the next time you run the program DRAGSTRIP PLUS will ask you if you wish to resume work on this vehicle, or start a new combination from scratch.

If you merely wish to access the vehicle database and do not wish to make either selection at the present time, press <ESC> or the <F8> key.  You will then be able to access and load a previously stored combination from the vehicle database.  The next time you restart DRAGSTRIP PLUS, you will be reminded once again of your unfinished vehicle combination.

## Vehicle Database

Press <F4> to select a vehicle record.

Press <F5>to copy a vehicle record.  This is a very useful feature which comes in handy if you have more than one configuration of your vehicle or just wish to experiment with your combination.  You can make your experimental changes to the copy rather than disturbing the original.

Press <F6> to swap two vehicle listings in the database.

Press <F7> to print the all database listings for your records.

Press <F9>to delete a single vehicle record.

Press <SHIFT> <F9>to delete a group of vehicle records.  [BE CAREFUL!]

Press <ESC> or <F8> to exit the vehicle database and return to DRAGSTRIP PLUS.

Press <ENTER> to page down through the database and see more vehicles (if any).

## Vehicle Database Information

The following information will mainly be of use to those users who are reasonably familiar with computers and DOS commands.

DRAGSTRIP PLUS stores your vehicle combinations in a disk file in the current directory.  This file can be distinguished from the other program files by its file extension, ".DB".

DRAGSTRIP PLUS' backup database file has a similar file extension, ".DBK".  Every time you enter a new vehicle combination into DRAGSTRIP PLUS or make any changes to an existing combination, the program saves your changes in its (".DB") database file.  In contrast, the backup database file (".DBK") stores your combinations as they were originally BEFORE you made your latest changes.

This knowledge can be very useful at times.  For example, if you delete a vehicle combination by mistake from the database while running the program, there is no way within DRAGSTRIP PLUS to get it back.  But if you were to were to exit DRAGSTRIP PLUS to DOS, and use the DOS copy command to copy the contents of the backup database file to the actual database file, your deleted vehicle combination (along with the rest) would be restored.

Here's an additional tip to our most advanced users:  You can prevent DRAGSTRIP PLUS from overwriting the backup database file, if you have a need to, by setting the DOS attribute flag of the backup database file to "Read Only".  This way you could permanently preserve a given set of vehicle combinations.

# Appendix  C

## Vehicle Weights

COMMON STREET RACE WEIGHTS FOR VARIOUS VEHICLES

VEHICLE	WEIGHT

Camaro, '67-69	3200

Camaro, 70's-90's	3400

Challenger	3400

Chevette, 8 cyl	2550

Chevy II Nova	3100

Corvette	3400

Falcon	3100

Fairlane	3000

GMC Silverado truck	4000

Grand National Regal	3400

Gremlin	3000

GT Mustang	3250

Malibu	3550

Mazda	2800

Mini truck	3400

Monte Carlo SS	3500

Nova	3400

T-bucket	2100

Trans Am	3500

Vega	2600

Ventura	3600

Volkswagon Beetle	1800

# Appendix  D

## Parameter File and Ratings File Documentary

FOR ADVANCED USERS AND PROGRAMMERS ONLY

YOU CAN MAKE DRAGSTRIP PLUS DO WHATEVER YOU WANT IT TO!

At initial start up, DRAGSTRIP PLUS looks for two files in the current directory.  The first is called "RACExxx.PRM", and the second is named "RACExxx.RTS".  If either file is missing, or discovered to be the wrong file length or is otherwise corrupted, DRAGSTRIP PLUS will recreate the offending file with a direct copy of its own internal default values.  The first file will hereafter be referred to as the "parameter file", while the second will be termed the "ratings file".

Both files are text (ASCII) files that can be easily examined or changed by DRAGSTRIP PLUS users as desired.  To make changes in either file, use a text file editor such as MS-DOS's "EDIT" or "EDLIN", or a word processor such as Professional Write or Word Perfect to edit the desired file.

The Ratings File.  The ratings file can be used to customize DRAGSTRIP PLUS'  E.T. and parts rating system to fit a particular specialized application, or to merely suit the user's taste.

For example, suppose a particular user constantly disagrees with DRAGSTRIP PLUS'  exhaust valve ratings (and related messages) in the Advice screen, thinking that DRAGSTRIP PLUS always gives exhaust valve ratings that are too high.

In this situation, DRAGSTRIP PLUS can be forced to change the way it rates all exhaust valves in the future by editing the ratings file and finding the following line:

0	Exhaust valve

By changing the "0" at the start of this line to a negative value, for example "-5", DRAGSTRIP PLUS will lower its ratings of all future exhaust values by this same amount (-5).  This line would then appear in the ratings file as follows:

-5	Exhaust valve

Of course, you can just as easily raise DRAGSTRIP PLUS' rating scheme for any part by changing the "0" at the start of the applicable line to a positive number, such as "5".

CAUTION:  Do not add or delete any lines to the ratings file, insert extra spaces in a line, or edit any portion of a line other than the numbers at the left side of the file, or change any rating (either up or down) by more than 30 points (or more than whatever value is stated at the top of the file).  DRAGSTRIP PLUS will check the file for gross errors, and will simply overwrite the ratings file with the original data if you make a critical error.  You may then try once again to modify the file to suit your application.  This process may be repeated as often as necessary, until you get it right.

However, keep in mind that DRAGSTRIP PLUS is most accurate calculating E.T.s when it is using the original ratings file; the program has not been seriously tested with other values, so results may be unpredictable.

The Parameter File.  The parameter file sets the program's parameter ranges by specifying a high and low value for every input value in the program.  The experienced user can change this file, in order to force the program to allow a wider range of input values in particular situations.

For example, supposed that the driver of a certain race car weighs 675 pounds. DRAGSTRIP PLUS normally rejects this input as being "out of range".  But if the parameter file is changed, DRAGSTRIP PLUS will accept a 675 pound driver without question.

Here's an example: in DRAGSTRIP PLUS, the first three input values in the program are MPH, Gear, and Tire Height (diameter) respectively.  Examining the first three sets of numbers in the parameter file reveals the low and high ranges for each in the following form (actual values you see may differ):

40	MPH

310

240	Gear

840

20	Tire Height

44

and continuing etc...

Note that the lowest permissible value DRAGSTRIP PLUS will accept for a given value always appears first, followed on the next line by its highest permissible value.

This means 40 - 310 is the only acceptable range for MPH input values, 2.40 to 8.40 is the valid Gear ratio input range, while 20 through 44 inches is the shortest and tallest tire heights allowed.

By examining the parameter file, you will be easily able to find the high and low parameter values controlling Driver's weight, 80 to 500 (pounds).  If you change this latter figure to 675 and save the file (if you are using a word processor, be sure to save the file in "TEXT" or "ASCII" format), from now on DRAGSTRIP PLUS will accept up to 675 pounds as a valid driver's weight, until you either change the parameter file again or delete it.

Note that some sets of values in the parameter file mean nothing, or are reserved for future use, and should be ignored.  These are clearly labeled in the parameter file as "Misc" or "Not used".

IMPORTANT: You can change any values you like, EXCEPT the high-low limits controlling values inputted into the program via a menu selection process.  Certain selections, such as "Tire Type", "Traction", "Manifold", and "Head Type", are made by the user by selecting the appropriate choice from a menu selection that appears during program execution.  Since you cannot make more menu selections appear other than those displayed (because no further menu choices exist), DRAGSTRIP PLUS considers any change to such parameter limits to be a fatal error, and will promptly overwrite your work with the original values (no harm done!).

Furthermore, you may not delete or add any parameters; when you finish modifying the parameter file, it must have the same number of low-value/ high-value parameter sets as it did originally.

As with the ratings file, there is no need to worry about making a mistake; DRAGSTRIP PLUS will check the file for gross errors, and your work will merely be overwritten with the original values if a serious error is discovered.  You may then attempt once more to try and modify the file to suit your application.  This process may be repeated as many times as necessary, until you get it right.

Also keep in mind that DRAGSTRIP PLUS is most accurate in calculating E.T.s when it is using its original parameter data; the program has not been seriously tested over wider parameter ranges, so the results may be quite unpredictable.  Particularly this may be so if the user modifies the MPH or Transmission Slip% ranges (and actually inputs data in this expanded range).

CAUTION: If you change certain parameter limits associated with the engine, such as "Engine Size" or "Carburetor CFM", this can have a radical effect (usually for the worse!) upon the E.T.s, MPHs, and/or shift points of every subsequent vehicle that is displayed, even previously saved "tried and true" combinations loaded from the program's database.

Changing the limits for other engine values such as "Intake Valve Size", may shift the program's rating system for that part (while correspondingly changing the range of any values you input, of course).  If you find this ratings change undesirable, change the ratings file (as described previously) until you correct the undesirable part rating shift.

The exact effects that result from changing any given value in the parameter file will not be documented in detail here, because these effects change from version to version of DRAGSTRIP PLUS as programming improvements are made.  You will have to gain this knowledge on your own, through experimentation and close observation.

If you modify the parameter and/or the ratings files in any way, DRAGSTRIP PLUS will display appropriate message(s) on the "E.T. RESULTS" screen advising caution against blindly trusting the program's E.T.  results.

If you wish to erase this message, or want to restore the parameter file and/or ratings file to their original state, simply delete the applicable file.  The next time you run DRAGSTRIP PLUS, the deleted file(s) will automatically be recreated (in whatever directory is current at the time) in their original form, and the caution message at the "E.T. RESULTS" screen will no longer appear (be careful not to delete any other DRAGSTRIP PLUS program files other than the parameter and/or ratings files).

# Appendix  E

## Setting Your Dial-In

In the section titled "Introducing DRAGSTRIP PLUS", the technique of using DRAGSTRIP PLUS to set your bracket racing "dial-in" was mentioned briefly.

Here is an example of how to do this:

Example:

If you get to the track late and don't have enough time for a trial run before bracket racing starts, you can use DRAGSTRIP PLUS to set your dial in, based upon your dial-in of the previous week.  Here's how:  Suppose the last time you raced the particular track you raced on was at an elevation of 785 feet, at an air pressure of 29.95, an air temperature of 70 degrees, a relative humidity of 45%, and your dial-in was 11.90.

First enter your combination into DRAGSTRIP PLUS exactly as it was the last time your raced it.  Suppose DRAGSTRIP PLUS predicts your E.T. to have been 11.77, which is slightly faster than its actual performance but within the program's tolerance range.

Now make changes as necessary to suit the new track's conditions: elevation 120 feet, air pressure 30.15, air temperature of 78 degrees, relative humidity 75%.

(TIP: use the database COPY function to make a copy of the original combination, if you wish to preserve the original combination unchanged).

Now suppose DRAGSTRIP PLUS predicts your E.T. to be 11.48.  Since DRAGSTRIP PLUS was too fast last week by .200 second, assume it will be too fast this week by .200 second again and add this same amount to its current prediction of 11.48, thereby arriving at 11.68.

Your dial-in for the new track would then be 11.68, down from the 11.90 the week before.

# Appendix  F

## Upcoming New Products

Revolutionary Concepts, Inc. is firmly committed to the continued evolution and improvement of DRAGSTRIP PLUS.  Meanwhile, Revolutionary Concepts' powerful professional version of DRAGSTRIP PLUS, entitled "DRAGSTRIP PLUS PROFESSIONAL", is currently under development and slated for release in mid 1996.

DRAGSTRIP PLUS PROFESSIONAL will store 1,000 vehicles instead of 100, include sophisticated vehicle database sort and query capabilities, include a library of over 1,000 popular camshafts, generate charts and graphics, and analyze more data than ever before.

DRAGSTRIP PLUS PROFESSIONAL will require a hard drive, VGA monitor, and 640K or more of RAM memory.  It is expected that DRAGSTRIP PLUS PROFESSIONAL will retail substantially above the price of DRAGSTRIP PLUS.

For more information about DRAGSTRIP PLUS PROFESSIONAL, or to obtain the latest version of DRAGSTRIP PLUS, please write or call:

Revolutionary Concepts, Inc.

P.O. Box 690048

Houston, Texas  77269-0048

Phone (713) 859-RACE

[collect return only]

Contact us via E-Mail for problems, lastest information or just to trade ideas:

dragplus@rconcepts.com